/* main_spi_eeprom_test2.c : spi peripheral */

//spi_eeprom_debug.ini ;

| /* * Simulate AT25 SPI EEPROM connected via SSP * AT25128: SPI EEPROM 128k (16k * 8) * Pins: * AT25xxx MCU * SCK - SCK (SSP) * SO - MISO (SSP) * SI - MOSI (SSP) * CS - P0.7 */ define unsigned char AT25_State; define unsigned long AT25_Address; define unsigned char AT25_Status; AT25_State = 0; AT25_Address = 0; AT25_Status = 0; MAP 0x10000000, 0x10003FFF READ WRITE // EEPROM func void AT25 (void) { switch (AT25_State) { case 0: // Instruction switch (SPI0_OUT & 0xFF) { case 0x06: // WREN if (AT25_Status & 0x01) break; AT25_Status ¦= 0x02; // Set Write Enable Bit break; case 0x04: // WRDI if (AT25_Status & 0x01) break; AT25_Status &= ~0x02; // Clear Write Enable Bit Break; | case 0x05: // RDSR SPI0_IN = AT25_Status; AT25_State = 1; break; case 0x01: // WRSR if (AT25_Status & 0x01) break; AT25_State = 2; break; case 0x03: // READ if (AT25_Status & 0x01) break; AT25_State = 3; break; case 0x02: // WRITE if (AT25_Status & 0x01) break; AT25_State = 4; break; } break; case 1: // Read SR SPI0_IN = AT25_Status; AT25_State = 0; break; case 2: // Write SR AT25_Status = (SPI0_OUT & 0x8C) ¦ (AT25_Status & ~0x8C); AT25_State = 0; break; case 3: // Get MSB Address for Read AT25_Address = (SPI0_OUT & 0x3F) << 8; AT25_State = 5; break; case 4: // Get MSB Address for Write AT25_Address = (SPI0_OUT & 0x3F) << 8; AT25_State = 6; break; |
| --- | --- |

| case 5: // Get LSB Address for Read AT25_Address ¦= SPI0_OUT & 0xFF; AT25_State = 7; SPI0_IN = _RBYTE(0x10000000 ¦ AT25_Address); break; case 6: // Get LSB Address for Write AT25_Address ¦= SPI0_OUT & 0xFF; AT25_State = 8; break; case 7: // Read AT25_Address = (AT25_Address + 1) & 0x3FFF; SPI0_IN = _RBYTE(0x10000000 ¦ AT25_Address); break; case 8: // Write if (AT25_Status & 0x02) { _WBYTE(0x10000000 ¦ AT25_Address, SPI0_OUT); AT25_Address = (AT25_Address & ~0x3F) ¦ ((AT25_Address + 1) & 0x3F); } break; } } // AT25 Chip Select Signal Function signal void AT25_CS (void) { while (1) { wwatch (PORT0); if (PORT0 & 0x00000080) { | if (AT25_State == 8) { AT25_State = 0; AT25_Status ¦= 0x01; // Set Busy Bit swatch (0.001); // Wait 1ms AT25_Status &= ~0x01; // Set Busy Bit } else { AT25_State = 0; } } } } // AT25 SPI Signal Function signal void AT25_SPI (void) { while (1) { wwatch (SPI0_OUT); twatch (1); if ((PORT0 & 0x00000080) == 0) AT25(); } } // Start AT25 Signal Functions AT25_CS() AT25_SPI() |
| --- | --- |