/* main_spi_eeprom_test1.c : spi bit banging*/

| #include <LPC21xx.h> #include "types.h" #include "delay.h" #include "defines.h" //defines for SPI Pin Function Select #define SCK 4 //P0.4 #define MISO 5 //P0.5 #define MOSI 6 //P0.6 #define CS 7 //p0.7 //25LC512 Instruction Set defines #define READ 0x03 #define WRITE 0x02 #define WRDI 0x04 #define WREN 0x06 #define sLED 16 //p1.16 #define fLED 17 //p1.17 u8 rDat __attribute__((at(0x40000000))); void Init_SPI(void) { //make CS as high SSETBIT(IOSET0,CS); //cfg CS as output SETBIT(IODIR0,CS); //make MOSI as high SSETBIT(IOSET0,MOSI) //cfg MOSI as output SETBIT(IODIR0,MOSI) //make clock line as high SSETBIT(IOSET0,SCK) //cfg clock line as output SETBIT(IODIR0,SCK); } u8 spi_eeprom_byteread(u16 rBuffAddr) { u8 rDat; //activate chip SCLRBIT(IOCLR0,CS); //spi transfer READ instruction to eeprom spi(READ); //spi transfer of msbyte of 16-bit address //rbuff address of byte to be read spi(rBuffAddr>>8); //spi transfer of lsbyte of 16-bit address // rbuff address of data to be read spi(rBuffAddr); //spi transfer of dummy byte to read byte // available at rBuffAddr into rData rDat=spi(0x00); //deactivate chip; SSETBIT(IOSET0,CS); return rDat; } main(void) { SETBIT(IODIR0,sLED); SETBIT(IODIR0,fLED); Init_SPI(); spi_eeprom_bytewrite(0,'B'); delay_ms(100); rDat=spi_eeprom_byteread(0); delay_ms(100); (rDat=='B') ? CPLBIT(IOPIN0,sLED): CPLBIT(IOPIN0,fLED); while(1); } | u8 spi(u8 sDat) { s32 i; u8 rDat; for(i=7;i>=0;i--) { SCLRBIT(IOCLR0,SCK); delay_us(1); READWRITEBIT2(IOPIN0,MOSI,sDat,i); delay_us(1); SSETBIT(IOSET0,SCK); delay_us(1); READWRITEBIT2(rDat,i,IOPIN0,MISO); delay_us(1); } //return recvd data return rDat ; } void WriteEnable(u8 cmd) { //activate chip; SCLRBIT(IOCLR0,CS) // spi transfer WREN command to eeprom; spi(WREN); //deactivate chip SSETBIT(IOSET0,CS); } void WriteDisable(u8 cmd) { //activate chip; SCLRBIT(IOCLR0,CS) // spi transfer WRDI command to eeprom; spi(WRDI); //deactivate chip SSETBIT(IOSET0,CS); } | void spi_eeprom_bytewrite(u16 wBuffAddr,u8 sData) { //activate write enable latch using write enable WriteEnable(); //activate chip SCLRBIT(IOCLR0,CS); //spi transfer WRITE instruction spi(WRITE); //spi transfer of msbyte of 16-bit address // wBuffAddr for data spi(wBuffAddr>>8); //spi transfer of lsbyte of 16-bit address // wbuff address for data spi(wBuffAddr); //spi transfer of sData for 16-bit //wbuff address spi(sData); //deactivate chip; SSETBIT(IOSET0,CS); //wait for internal write cycle time delay of 5 to 10ms; delay_ms(10); //deactivate write enable latch using write disable WriteDisable(); } |
| --- | --- | --- |