| /* main_spi_eeprom_test_2.c : spi peripheral driver */ #include <LPC21xx.h> #include "types.h" #include "delay.h" #include "defines.h" //define CFGPIN macro #define CFGPIN(WORD,PIN,FUNC) \ WORD=((PIN<16) ? \ ((WORD&~(3<<(PIN*2)))¦ \ (FUNC<<(PIN*2))) : \ ((WORD&~(3<<((PIN-16)*2)))¦ \ (FUNC<<((PIN-16)*2)))) //defines for SPI Pin Function Select #define FUNC2 1 // #define SCK FUNC2 //P0.4 #define MISO FUNC2 //P0.5 #define MOSI FUNC2 //P0.6 #define CS 7 //p0.7 // S0SPCR Bits Setting #define Mode_0 0x00 // CPOL 0 CPHA 0 #define Mode_1 0x08 // CPOL 0 CPHA 1 #define Mode_2 0x10 // CPOL 1 CPHA 0 #define Mode_3 0x18 // CPOL 1 CPHA 1 #define MSTR_BIT 5 // 1=Master,0=Slave #define LSBF_BIT 6 // 0=MSB first,1=LSB first // S0SPSR Status Register bits #define SPIF_BIT 7 // Data Transfer Completion Flag //25LC512 Instruction Set defines #define READ 0x03 #define WRITE 0x02 #define WRDI 0x04 #define WREN 0x06 #define sLED 16 #define fLED 17 u8 rDat __attribute__((at(0x40000000))); void Init_SPI(void) { //cfg p0.4 pin as SCK pin using PINSEL0 CFGPIN(PINSEL0,SCK,FUN2); //cfg p0.5 pin as MOSI pin using PINSEL0 CFGPIN(PINSEL0,MOSI,FUN2); //cfg p0.6 pin as MISO pin using PINSEL0 CFGPIN(PINSEL0,MISO,FUN2); //make p0.7(cs) as high using IOSET0/IOPIN0 SETBIT(IOPIN0,CS); //cfg p0.7 as gpio output pin using IODIR0 SETBIT(IODIR0,CS); //cfg spi clock speed using S0SPCCR S0SPCCR=60; //cfg spi peripheral for Master Mode & Mode3 S0SPCR=1<<MSTR_BIT¦Mode_3; } u8 spi_eeprom_byteread(u16 rBuffAddr) { u8 rDat; //make chip select low SCLRBIT(IOCLR0,CS); //issue READ instruction spi(READ); //msbyte of 16-bit address loc to be read via spi spi(rBuffAddr>>8); //lsbyte of 16-bit address loc to be read via spi spi(rBuffAddr); //sending garbage & reading data at loc rDat=spi(0x00); //make chip select high SSETBIT(IOSET0,CS); return rDat; } | u8 spi(u8 sDat) { //load sDat into S0SPSR S0SPDR=sDat; // wait for transmission to complete //using S0SPSR and monitoring SPIF bit while(READBIT(S0SPSR,SPIF_BIT)==0); // return read data from S0SPDR; return S0SPDR; } void WriteEnable(void) { //make chip select low SCLRBIT(IOCLR0,CS); //issue WREN via spi tranfer spi(WREN); //make chip select high SSETBIT(IOSET0,CS); } void WriteDisable(void) { //make chip select low SCLRBIT(IOCLR0,CS); //issue WRDI via spi transfer spi(WRDI); //make chip select high SSETBIT(IOSET0,CS); } void spi_eeprom_bytewrite(u16 wBuffAddr,u8 sDat) { //Enable Write Enable Latch WriteEnable(); //make chip select low SCLRBIT(IOCLR0,CS); //issue WRITE instruction via spi transfer spi(WRITE); //msbyte of 16-bit address loc via spi transfer spi(wBuffAddr>>8); //lsbyte of 16-bit address loc via spi transfer spi(wBuffAddr); //data for that address loc spi(sDat); //make chip select high SSETBIT(IOSET0,CS); //delay for internal write cycle delay_ms(10); //Disable Write Enable Latch WriteDisable(); } main(void) { SETBIT(IODIR1,sLED); SETBIT(IODIR1,fLED); Init_SPI(); spi_eeprom_bytewrite(0,'*'); delay_ms(100); rDat=spi_eeprom_byteread(0); delay_ms(100); (rDat=='*')?CPLBIT(IOPIN1,sLED):CPLBIT(IOPIN1,fLED); while(1); } |
| --- | --- |