Question Bank CISC vs RISC

- In __________ architecture  Instructions are of fixed size.

- In __________ architecture instructions have varying sizes, & varying fetching & execution times/cycles.

- In __________ architecture instruction set is simple and small.

- The following are true about CISC architecture except.

- Execution speed is slower as most operations are memory based.

- Ideal for processors performing a variety of operations.

- These generally require micro-programmed control unit

- None of the above

- The following are true about RISC architecture except.

- Use Instructions of fixed size

- Most Instructions take same time (1 cycle) to execute except for a few.

- Lesser Addressing Modes

- Total size of program is small as few instructions are required to perform a task.

- RISC architectures are ideal for processor performing a dedicated operation. (True/False)

- For CISC architectures since number of cycles per instruction varies, pipelining has ___________ (more/less) bubble or stalls.

- _____________ (CISC/RISC) architecture give a better degree of pipelining over _____________(CISC/RISC) architecture.

- RISC processors employ _____________ instruction set model , since all ALU operations will be performed on register operands ONLY and not on memory locations.

- _______________ instruction set model in RISC eliminates having multiple types of the same instructions and hence _________ the total number of instructions.

- RISC processors usually have _________ core registers compared to __________ processors.

- RISC processors consume __________ power because most operations are register based.

- CISC processors are power hungry. (True/False)

- In __________ architectures, since number of cycles per instruction varies, pipelining has more bubbles or stalls or lags.

- There can be no pipelining stalls in RISC architectures. (True/False)

- In __________ architectures, since number of cycles per instruction is mostly fixed, except for a few, it gives a better degree of pipelining.

- In CISC architectures ______________________________________________.

- Pipelining has more bubbles or stalls or lags.

- Have a better degree of pipelining.

- RISC architectures relatively support _________________________ as most operations are register based.

- Less addressing modes

- Complex addressing modes

- Same time to fetch instructions

- Different fetching times

- Compiler design is relatively________ for RISC architectures.

- Compiler design is relatively________ for CISC architectures.