LPC2129 uC & Pin Connect Block

- The LPC2129 uC belongs to the family labelled as __________ series .

- The ____________ series is a series of microcontroller ICs designed using the ARM7 Core, with the processor/implementation having the nomenclature _____________.

- The LPC2129 uC is ________ pin ___________ package, i.e. a square shaped IC, each with ________ pins on each side.

- In LPC2129 uC _____ pins out of ______ pins are available for I/O operations ,which are segregated into _____ ports , namely _______ & _______.

- The port 0 has about ______ pins ,whereas  port 1 has ______pins hence accounting for the total of ______ I/O pins out of ______ pins.

- Each of the ____ I/O pins can be configured for______ functions BUT ONLY one function at a point of time can be used for any application ,depending on the embedded application’s need.

- The ________________ peripheral is used for configuring the functionality of any of the 46 I/O pins for a particular function.

- The Pin Connect Block Peripheral has _____ SFRs ,namely _______________, ______________, _____________.

- Each of the PINSELx SFR is  _____ bit, with _______ bits required to configure functionality of any  particular pin among the available 46 I/O pins.

- The values ranging between ____ to ____ can be used to configure the bit pair for selecting any pin function.

- The PINSEL0 SFR supports the pin configuration for pins label between _____ to_____.

- The PINSEL1 SFR supports the pin configuration for pins label between _____ to_____.

- The PINSEL2 SFR supports the pin configuration for pins label between _____ to_____.

- The common function of any of the 46 I/O pins is _________.

- The ____________ header file contains the declarations for PINSELx SFRs in the KEIL IDE.

- The default functions of most of the 46 I/O pins is ________.

- Port ___ pins do not have alternate functions other than GPIO.

- Pin labelled from ____ to ____ do not have alternate functions other than GPIO.

- Assume previous configuration for P0.0 is function 1/3, to configure the P0.0 pin for function 2, without affecting other pin functions, ___________________among the following Embedded C statements is the best option.

- PINSEL0 |=2 ;

- PINSEL0 |=0x00000002;

- PINSEL0 |= 2<<0 ;

- PINSEL0 = (PINSEL0 & ~(3<<(0*2))) | (2<<(0*2)) ;

- To configure the P0.0 pin ,for function 3,with default function as GPIO & without affecting other pin functions, _____________________among the following Embedded C statements is the best  option.

- PINSEL0 |=3 ;

- PINSEL0 |=0x00000003;

- PINSEL0 |= 3<<0 ;

- Any of the above ;

- Given the Embedded C Argumented Macro with its expansion below

#define CFGPIN(WORD,PIN,FUNC) \

(WORD=(PIN<16) ? \

(WORD&~((unsigned)3<<(PIN*2))|(FUNC<<(PIN*2))) : \

(WORD&~((unsigned)3<<((PIN-16)*2))|(FUNC<<((PIN-16)*2))))

To configure the P1.16 pin ,for function 3,with default function as GPIO & without affecting other pin functions, _____________________among the following Embedded C statements is the best  option.

- CFGPIN(PINSEL0,0,3) ;

- CFGPIN(PINSEL1,16,3)

- CFGPIN(PINSEL2,16,3)

- None of the above

- To configure the P0.0 pin ,for function 3,irrespective/unaware of its current function & without affecting other pin functions, option ____ Assembly Language statements are the best  option.

PINSEL0 EQU 0xE002C000

AREA RESET,CODE,READONLY

ARM

ENTRY

START

;CHOOSE UR OPTION TO ADD UR INSTRUCTIONS HERE

HERE     B HERE

END

- LDR R0,=PINSEL0;

MOV R1,#3

STR R1,[R0]

- LDR R0,=PINSEL0;

LDR R1,[R0]

ORR R1,R1,#3

STR R1,[R0]

- LDR R0,=PINSEL0;

LDR R1,[R0]

BIC R1,R1,#3

ORR R1,R1,#3

STR R1,[R0]

- LDR R0,=PINSEL0;

LDR R1,[R0]

MOV R2,#3

MOV R3,#-1

EOR R2,R2,R3

AND R1,R1,R2

ORR R1,R1,#3

STR R1,[R0]

- The declarations/prototypes for software delay generation routines are

- void delay_us (unsigned int dlyUS);

- void delay_ms(unsigned int dlyMS);

- a or b

- a & b

- The definition for software delay generation routine for delays in the order of “us”  can be

- void delay_us (unsigned int dlyUS) {  dlyUS*=12; while(dlyUS--); }

- void delay_us (unsigned int dlyUS) {  dlyUS*=12000; while(dlyUS--); }

- void delay_us (unsigned int dlyUS) {  dlyUS*=12000000; while(dlyUS--); }

- None of the above

- The definition for software delay generation routine for delays in the order of “ms”  can be

- void delay_ms (unsigned int dlyMS) {  dlyUS*=12; while(dlyMS--); }

- void delay_ms (unsigned int dlyMS) {  dlyUS*=12000; while(dlyMS--); }

- void delay_ms (unsigned int dlyMS) {  dlyUS*=12000000; while(dlyMS--); }

- None of the above

- The definition for software delay generation routine for delays in the order of “s”  can be

- void delay_s (unsigned int dlyS) {  dlyUS*=12; while(dlyS--); }

- void delay_s (unsigned int dlyS) {  dlyUS*=12000; while(dlyS--); }

- void delay_s (unsigned int dlyS) {  dlyUS*=12000000; while(dlyS--); }

- None of the above

- The Embedded C syntax to reserve & initialize globally,the Flash ROM, with  a string “Embedded C Programming” is

- const char s[]=”Embedded C Program”;

- const char s[] __attribute__((at(0x00003000)))="Embedded C Programming";

- a or b

- None of the above

- For the code given below

#include <LPC21xx.h>

#define FUNC1 0

#define FUNC2 1

#define FUNC3 2

#define FUNC4 3

#define CFGPIN(WORD,PIN,FUNC) \

(WORD=(PIN<16) ? \

(WORD&~((unsigned)3<<(PIN*2))|(FUNC<<(PIN*2))) : \

(WORD&~((unsigned)3<<((PIN-16)*2))|(FUNC<<((PIN-16)*2))))

int main()

{

//add ur statement(s) to config P0.0 for function 3 & config P0.7 for function 2 without using    CFGPIN Macro

//add ur statement to config P0.15 for function 4 using CFGPIN Macro

//add ur statement to config P0.27 for function 1 without using CFGPIN Macro

while(1);

}

- The Embedded C syntax to reserve globally 30 bytes of RAM to accommodate some string ,which the firmware/bare-metal program shall use later during execution/runtime.

- char s[30] ;

- char str[30] __attribute__((at(0x40000000)));

- a or b

- None of the above

- The Embedded C syntax to reserve & initialize globally,the Flash ROM, with  a string “Embedded C Programming” & reserve globally 30 bytes of RAM to accommodate the string read from Flash Memory to RAM ,which the firmware/bare-metal program shall do, during execution/runtime is

- const char s[]=”Embedded C Program”;

char s[30] ;

- const char s[] __attribute__((at(0x00003000)))="Embedded C Programming";

char str[30] __attribute__((at(0x40000000)));

- a or b

- None of the above

- The Assembly  syntax to reserve & initialize the Flash ROM, with  a string

“Embedded C Programming” & reserve 30 bytes of RAM to accommodate the string read from  Flash Memory to RAM ,which the firmware/bare-metal program shall do, during execution/runtime is

- AREA ROM_STRINGS,CODE,READONLY

SRC_STRING1 DCB ”Embedded C Program”,’\0’,’\0’

- AREA RAM_STRINGS,DATA,READWRITE

DST_STRING DS 30

- a or b

- a & b

- The Assembly  syntax to reserve & initialize 30 bytes the Flash ROM, with 0x45.

AREA ROM_BYTES,CODE,READONLY

ROM_DATA FILL ____,_____,_____