LPC2129 uC & GPIO Peripheral Block

- The GPIO Peripheral is  ___________ peripheral.

- On Chip User

- On Chip System

- Off Chip User

- Off Chip System

- The GPIO Peripheral Block has ______ SFRs.

- 4

- 6

- 8

- 10

- In The GPIO Peripheral Block has SFRs of each _____bit wide .

- 8

- 16

- 24

- 32

- The GPIO Block has SFRs for

- Configuring the direction of  I/O pin(s)

- Setting / Clearing the I/O pin(s) exclusively, without affecting other pins

- Manipulation I/O pin(s) ,with programmer taking care, other pins should not be affected

- Reading the Status of I/O pin(s).

- All of the above

- Map the GPIO Block has SFRs for with their Use

USE                                                                                                            SFR

- Configuring the direction of  I/O pin(s)                                              IOPINx

- Setting / Clearing the I/O pin(s) ,                                                        IODIRx

Hardware ensures without affecting other pins

- Manipulation I/O pin(s) ,with programmer taking care,                 IOSETx

other pins should not be affected

- Reading the Status of I/O pin(s).                                                         IOCLRx

- The Assembly Language MACRO SETBIT WORD,BITPOS can be used for ____________ SFR(s).

- IODIRx

- IOSETx

- IOCLRx

- All of the above

- The Assembly Language MACRO CLRBIT WORD,BITPOS cannot be used for ____________ SFR(s).

- IODIRx

- IOSETx

- IOPINx

- All of the above

- For the given macro expansion  “(WORD&~((unsigned)3<<(PIN*2))|(FUNC<<(PIN*2)))” which is the appropriate Embedded C Macro ?

- #define SETBIT(WORD,BITPOS)

- #define CLRBIT(WORD,BITPOS)

- #define CPLBIT(WORD,BITPOS)

- None of the above

- For the given macro expansion  “WORD=((WORD&~(255<<BITPOS))|(BYTE<<BITPOS))” which is the appropriate Embedded C Macro ?

- #define SETBIT(WORD,BITPOS)

- #define CLRBIT(WORD,BITPOS)

- #define CPLBIT(WORD,BITPOS)

- None of the above

- For the given macro expansion  “WORD=(1<<BITPOS)” which is the appropriate Embedded C Macro ?

- #define SETBIT(WORD,BITPOS)

- #define CLRBIT(WORD,BITPOS)

- #define CPLBIT(WORD,BITPOS)

- None of the above

- For the given macro expansion  “WORD=~(1<<BITPOS)” which is the appropriate Embedded C Macro ?

- #define SETBIT(WORD,BITPOS)

- #define CLRBIT(WORD,BITPOS)

- #define CPLBIT(WORD,BITPOS)

- None of the above

- For the given macro expansion  “WORD^=(1<<BITPOS)” which is the appropriate Embedded C Macro ?

- #define SETBIT(WORD,BITPOS)

- #define CLRBIT(WORD,BITPOS)

- #define CPLBIT(WORD,BITPOS)

- None of the above

- Map the Embedded C Macros on LHS with their appropriate expansion on RHS?

Macro                                                                                                 Expansion

- #define SETBIT(WORD,BITPOS)                                      WORD&=~(1<<BITPOS)

- #define READBIT(WORD,BITPOS)                                   ( (WORD>>BITPOS)&255)

- #define CLRBIT(WORD,BITPOS)                                      WORD|=(1<<BITPOS)

- #define READBYTE(WORD,BITPOS)                               WORD^=(1<<BITPOS)

- #define CPLBIT(WORD,BITPOS)                                      ( (WORD>>BITPOS)&1)

- #define WRITEBYTE(WORD,BITPOS,BYTE)      WORD=((WORD&~(255<<BITPOS))|(BYTE<<BITPOS))

- To read the status of a bit from some word & update the same into another word which Embedded C  Macro options can be used ?

- READBIT(WORD,BITPOS) , WRITEBIT(WORD,BITPOS,BIT)

- WRITEBIT(WORD,BITPOS,READBIT(WORD,BITPOS)) //nested macros

- READWRITEBIT(DWORD,BITPOS,SWORD,BITPOS)

- Any one of the above

- The Macro Expansion “WORD=((WORD&~(15<<STARTBITPOS))|(VAL<<STARTBITPOS)) is for the Macro

- READBIT(WORD,____ )

- READNIBBLE(WORD, _____)

- READBYTE(WORD, ____)

- None of the above

- To  Serialize Out a Byte Data, MSB to LSB order, via Port 0 Pin 0 ( P0.0) to the external world @100ms/bit ,which Embedded C Macro will fit the “add ur statement here” in the program given below ?

#include <LPC21xx.h>

#include “types.h”

#include “delay.h”

#include “defines.h”

#define OPIN_0_0   0    //p0.0

int main()

{

s8 i;

u8 sDat=’A’ ;

SETBIT(IODIR0,OPIN_0_0);

for (i=7;i>=0; i--)

{

‘’add ur statement(s) here” ; //to serialize out via P0.0

delay_ms(100);

}

}

- WRITEBIT(IOPIN0,OPIN_0_0,((sDat>>i)&1))

- READWRITEBIT2(IOPIN0,OPIN_0_0,sDat,i);

- A or B

- A & B

- To  Serialize Out a Byte Data, LSB to MSB order, via P0.0 to the external world @100ms/bit ,which Embedded C Macro will fit the “add ur statement here” in the program given below ?

#include <LPC21xx.h>

#include “types.h”

#include “delay.h”

#include “defines.h”

#define OPIN_0_0   0    //p0.0

int main()

{

s8 i;

u8 sDat=’A’ ;

SETBIT(IODIR0,OPIN_0_0);

for (i=0;i<=7; i++)

{

‘’add ur statement(s) here” ; // to serialize out via P0.0;

delay_ms(100);

}

}

- WRITEBIT(IOPIN0,OPIN_0_0,(sDat>>i)&1)

- READWRITEBIT2(IOPIN0,OPIN_0_0,sDat,i);

- A or B

- A & B

- To  Serialize In a Byte Data, MSB to LSB order, via P0.0 from the external world @100ms/bit  & store in an unsigned char type variable, which Embedded C Macro will fit the “add ur statement(s) here” in the program given below ?

#include <LPC21xx.h>

#include “types.h”

#include “delay.h”

#include “defines.h”

#define IPIN_0_0   0    //p0.0

int main()

{

s8 i;

u8 rDat=0 ;

CLRBIT(IODIR0,IPIN_0_0);

for (i=7;i>=0; i--)

{

delay_ms(100);

‘’add ur statement(s) here” ; // to serialize in via P0.0;

}

}

- READBIT(IOPIN0,IPIN_0_0) ? SETBIT(rDat,i) : CLRBIT(rDat,i);

- READWRITEBIT2(rDat,i,IOPIN0,IPIN_0_0);

- WRITEBIT(rDat , i, READBIT(IOPIN0,IPIN_0_0))

- Any One of the above

- None of the above

- To  Serialize In a Byte Data, LSB to MSB order, via P0.0 from the external world @100ms/bit  & store in an unsigned char type variable, which Embedded C Macro will fit the “add ur statement(s) here” in the program given below ?

#include <LPC21xx.h>

#include “types.h”

#include “delay.h”

#include “defines.h”

#define IPIN_0_0   0    //p0.0

int main()

{

s8 i;

u8 rDat=0 ;

CLRBIT(IODIR0,IPIN_0_0);

for (i=i;i<=7; i++)

{

delay_ms(100);

‘’add ur statement(s) here” ; // to serialize in via P0.0;

}

}

- READBIT(IOPIN0,IPIN_0_0) ? SETBIT(rDat,i) : CLRBIT(rDat,i);

- READWRITEBIT2(rDat,i,IOPIN0,IPIN_0_0);

- WRITEBIT(rDat , i, READBIT(IOPIN0,IPIN_0_0))

- Any One of the above

- None of the above

- Which among the following Embedded Statement which would be optimized to set pin 7 of port 0 (p0.7) ?

- IOSET0=1<<7;

- IOSET0|=1<<7;

- IOPIN0|=1<<7;

- SETBIT(IOSET0,7)

- SETBIT(IOPIN0,7);

- WRITEBIT(IOSET0,7,1);

- WRITEBIT(IOPIN0,7,1);

- Which among the following Embedded Statement which would be optimized to clear pin 7 of port 0 (p0.7) ?

- IOCLR0=1<<7;

- IOCLR0|=1<<7;

- IOPIN0&=~(1<<7);

- SETBIT(IOCLR0,7)

- CLRBIT(IOPIN0,7);

- WRITEBIT(IOCLR0,7,1);

- WRITEBIT(IOPIN0,7,0);

- Which among the following Embedded Statement which would be optimized to toggle pin 7 of port 0 (p0.7) ?

- IOPIN0&=~(1<<7);

- CPLBIT(IOPIN0,7);

- u8 bit=0; WRITEBIT(IOPIN0,7,(bit=!bit));

- A or B

- Which among the following Embedded C Macros would be suitable for Statement given below

“((IOPIN0>>31)&1)”;  ?

- SETBIT(IOPIN0,31)

- CLRBIT(IOPIN0,31)

- CPLBIT(IOPIN0,31);

- None of the above

- Which among the following Embedded C Macros would be suitable for Statement given below

“((IOPIN0&~(255<<24))|(0x55<<24)”;  ?

- SETBIT(IOPIN0,24);

- CLRBIT(IOPIN0,24)

- CPLBIT(IOPIN0,24);

- None of the above

- Which among the following Embedded C Macros would be suitable for Statement given below

“((IOPIN0&~(15<<24))|(15<<24)”;  ?

- SETBIT(IOPIN0,24);

- CLRBIT(IOPIN0,24)

- CPLBIT(IOPIN0,24);

- None of the above

- Write the appropriate statements in place of the “add ur assembly statement” in the code given below to blink an active low LED  at p0.7 @100ms infinitely.

;FILE : LED_BLINK_USING_MACROS_SUBROUTINES_IN_SINGLE_FILE_1.S

; REQUIRES TO BE ASSEMBLED ALONG WITH (MODIFIED) Startup.s

; FINAL EXECUTABLE IS HEX FILE TO FLASHED INTO ON-CHIP ROM

PINSEL0  	EQU      0xE002C000

IOPIN0   	EQU      0xE0028000

IODIR0   	EQU      0xE0028008

LED         	EQU      7

AREA RESET,CODE,READONLY

ARM

ENTRY

EXPORT __main ;  linkage for modified Startup.S

MACRO   ; Macro definition WRITING BIT 1/0 INTO ANY SFR BITPOS

WRITEBIT $WORDVAR,$BITPOS,$BITLEVEL

;ADD UR STATEMENT HERE FOR R0 TO HOLD MEM ADDR

;ADD UR STATEMENT HERE FOR R1 TO HOLD BIT POSITION TO WRITE

;ADD UR STATEMENT HERE FOR R2 TO HOLD BIT LEVEL TO WRITE

BL SR_WRITEBIT

MEND

MACRO   ;MACRO DEFINITION FOR DELAY_MS

DELAYMS $DLYMS

LDR R0,= $DLYMS

;ADD UR STATEMENT TO CALL DELAYMS SUBROUTINE

MEND

__main               ;control branches from modified Startup.s file to this point in this file

;ADD UR STATEMENT TO CFG BIT 14 OF PINSEL WITH BIT LEVEL FOR P0.7 AS GPIO FUNC

;ADD UR STATEMENT TO CFG BIT 15 OF PINSEL WITH BIT LEVEL FOR P0.7 AS GPIO FUNC

;ADD UR STATEMENT TO CFG BIT 7 OF IODIR0 FOR P0.7 AS GPIO OUTPUT PIN LEVEL 0

AGAIN

;ADD UR STATEMENT TO MAKE P0.7 PIN HIGH

DELAYMS 100

;ADD UR STATEMENT TO MAKE P0.7 PIN HIGH

DELAYMS 100

B AGAIN

;SUB-ROUTINES FOR WRITING BIT '0/1' TO BITPOS IN MEMORY

SR_WRITEBIT

LDR R3,[R0]      ;GET WORD FROM MEMORY INTO R3

MOV R4,#1        ;

;ADD UR STATEMENT HERE ;1<<BITPOS

CMP R2,#1        ;CHECK BIT LEVEL TO WRITE '1' OR '0'

;ADD UR STATEMENT HERE   ;IF BIT LEVEL '1' WRITE IN R3|=R4

;ADD UR STATEMENT HERE   ;IF BIT LEVEL '0' WRITE IN R3&=~R4

STR R3,[R0]      ;UPDATE WORD BACK INTO MEMORY

MOV PC,LR        ;EXIT SUB-ROUTINE SR_WRITEBIT

SR_DELAYMS ;BEGIN DELAYMS SUBROUTINE

LDR R1,=12000     ;~1MS DELAYMS IN R0 @ 60MHz

;ADD UR STATEMENT HERE   ;N * 1MS

DELAYMS_L1

CMP R2,#0        ;dlyMS>0

;ADD UR STATEMENT HERE  ;CONDITIONAL EXECUTION for dlyMS--

BHI DELAYMS_L1   ;CONDITIONAL BRANCHING TO L1

;ADD UR STATEMENT HERE     ;EXIT DELAYMS SUBROUTINE

;SUB-ROUTINES FOR SETTING BIT IN BITPOS IN MEMORY

END