;SWITCH_LED_MACROS_SUBROUTINES_SINGLE.S

;ACTIVE LOW SWITCH & ACTIVE LOW LED

;REQUIRES myStartup.s (MODIFIED Startup.s FILE)

PINSEL0  EQU 0xE002C000

IOPIN0   EQU 0xE0028000

IODIR0   EQU 0xE0028008

SW_AL    EQU 0 ;SW  @PIN 0 OF PORT0

LED_AL   EQU 7 ;LED @PIN 7 OF PORT0

AREA MYAPP,CODE,READONLY

ARM

ENTRY

EXPORT __main

MACRO   ; Macro definition WRITING BIT INTO ANY SFR

WRITEBIT $WORDVAR,$BITPOS,$BITLEVEL

LDR R0,= $WORDVAR

MOV R1,#$BITPOS

MOV R2,#$BITLEVEL

;ADD UR INSTRUCTION TO USE WRITEBIT SUB-ROUTINE

MEND

MACRO   ; Macro definition READING BIT FROM ANY SFR

READBIT $WORDVAR,$BITPOS

LDR R0,= $WORDVAR

MOV R1,#$BITPOS

;ADD UR INSTRUCTION TO USE READBIT SUB-ROUTINE

MEND

;MAIN/CALLING BLOCK

__main

;ADD UR INSTRUCTION TO CFG LED PIN AS GPIO OUT PIN USING  MACRO

AGAIN

;ADD UR INSTRUCTION TO GET SWITCH PIN STATUS USING MACRO

;ADD UR INSTRUCTION TO CHECK SWITCH PIN STATUS AS PRESSED/RELEASED

BEQ LED_ON

BNE LED_OFF

LED_OFF	;ADD UR INSTRUCTION TO MAKE LED OFF USING MACRO

B AGAIN

LED_ON 		;ADD UR INSTRUCTION TO MAKE LED ON USING MACRO

B AGAIN

;END OF MAIN/CALLING BLOCK

SR_WRITEBIT	LDR R3,[R0]

MOV R4,#1

MOV R4,R4,LSL R1

CMP R2,#1

ORREQ R3,R3,R4

BICNE R3,R3,R4

STR R3,[R0]

MOV PC,LR

SR_READBIT	LDR R2,[R0]

MOV R2,R2,LSR R1

AND R2,R2,#1

MOV PC,LR

END