INTERRUPTS/EXCEPTIONS

- ARM 7 TDMI-S Processor support __________ sources of interrupts.

- Interrupt refers _________ / event / disturbance that breaks on the on-going flow of CPU execution to branch/divert/redirect/jump to another flow of execution in response to the interrupt raised.

- The respective block of statements/instructions which must be executed in response to the interrupt raised is referred to by abbreviation as ISR which can synonymously called as ________________________________ /______________________________ /______________________.

- The starting address of  different ISRs for their respective interrupts are termed as _____________________ .

- The interrupt vector(s) are usually _______________ for  CPU architecture & may sometimes be remapped to  other memory areas/locations.

- A memory area/location which is supposed to have the set/array of interrupt vectors is referred to as __________________________________.

- In the Startup.s  file generated during project creation in Keil IDE for LPC2129 has the instruction type __________ being used to fill the Interrupt Vector Table (IVT).

- The ARM7TDMI-S Processor has _______ sources of interrupts applied via interrupt input lines namely nFIQ & nIRQ.

- The LPC2129 uC has an on-chip interrupt controller name as _____________________________abbreviated as _____.

- The VIC has  __________  SFRs which can be looked up via the datasheets or the LPC21xx.h header file.

- The External Interrupt Peripheral has _______ SFRs which can be looked up via the datasheets or the LPC21xx.h header file.

- In LPC2129 uC the port pins which can be used alternately for External Interrupt Input pins/lines for External Interrupt Peripheral are ______,______, _______, _______ for EINT0,EINT1,EINT2 & EINT3 respectively.

- In LPC2129 uC the port pins to be used alternately for External Interrupt Input pins/lines   EINT0,EINT1,EINT2 & EINT3  their respective  bit pairs have to be configured for _________ function having value _____ using PINSEL0 SFR of the PIN CONNECT BLOCK.

- The syntax for  IRQ Type Interrupt Declarations/Prototypes using Embedded C Cross Compiler from KEIL IDE is _________________________________.

- The ________________________ instruction available at Interrupt Vector 0x00000018 in Startup.s transfers the control of CPU execution to user defined ISR of type IRQ.

- __________________________ SFR(s) need to be configure for to allow ANY one of the 4 External Interrupts to be channel via  VIC as Vectored IRQ with priority level 0.

- _______________ SFR is used for configuring the External Interrupt(s) for the triggering level as Falling/Rising Edge

- The VIC in LPC2129 uC can allow upto a maximum of _______ interrupt request inputs.

- For the Code Given Below Replace the “add ur statement No here”  with appropriate statement.

/* external_interrupt_test.c : USING MACROS */

#include <lpc21xx.h>

#define FUNC1 0

#define FUNC2 1

#define FUNC3 2

#define FUNC4 3

#define CFGPIN(WORD,PIN,FUNC) \

WORD=((PIN<16) ? \

((WORD&~(3<<(PIN*2)))|(FUNC<<(PIN*2))): \

((WORD&~(3<<((PIN-16)*2)))|(FUNC<<((PIN-16)*2))));

#include "defines.h"

#define SETBIT(WORD,BITPOS)         	(WORD|=(1<<BITPOS))

#define CLRBIT(WORD,BITPOS)         	(WORD&=~(1<<BITPOS))

#define CPLBIT(WORD,BITPOS)         	(WORD^=(1<<BITPOS))

#define SSETBIT(WORD,BITPOS) 	WORD=1<<BITPOS

#define SCLRBIT SSETBIT

#define EINT0_LED 16                                 //EINT0 Status LED connected @ p0.16

//add ur statement 1 here  // declaration for external interrupt 0 interrupt service routine;

//add ur statement 2 here  // declaration for Activating External Interrupt 0;

unsigned int gCnt;

main()

{

//add ur statement 3 here;  //To Activate External Interrupt 0

while(1) {   gCnt++;   }

}

void Enable_EINT0(void)

{

//add ur statement 4 here; //to cfg p0.1 pin as EINT0 interrupt input pin using PINSEL0 SFR

//add ur statement 5 here; //to cfg EINT0 status LED pin as output pin using IODIR0 SFR

//add ur statement 6 here; //to enable External Interrupt 0 in VIC using VICIntEnable SFR

//add ur statement 7 here; //to enable External Interrupt 0 as Vector IRQ with priority 0 (highest)                                               			     //using VICVecCntl0 SFR

//add ur statement 8 here; //to write the External Interrupt 0’s ISR address into 					                    //VICVectAddr0

//add ur statement 9 here; // to enable EINT0 using EXTINT0 SFR

//add ur statement 10 here; //to cfg EINT0 as EDGE_TRIG using EXTMODE

//add ur statement 11 here; //to cfg EINT0 for RISING EDGE using EXTPOLAR

}

void eint0_isr(void) __irq

{

//add ur statement 12 here; //to toggle EINT0 status LED as confirmation of isr activity in action

// add ur statement 13 here; //to clear flag EINT0 status flag using EXTINT SFR

//add ur statement 14 here;  //to perform dummy write operation into VICVectAddr

}

- For the Code Given Below Replace the “add ur statement No here”  with appropriate statement.

/* external_interrupt_test.c : WITHOUT USING ANY MACROS */

#include <lpc21xx.h>

//add ur statement 1 here  // declaration for external interrupt 0 interrupt service routine;

//add ur statement 2 here  // declaration for Activating External Interrupt 0;

unsigned int gCnt;

main()

{

//add ur statement 3 here;  //To Activate External Interrupt 0

while(1) {   gCnt++;   }

}

void Enable_EINT0(void)

{

//add ur statement 4 here; //to cfg p0.1 pin as EINT0 interrupt input pin using PINSEL0 SFR

//add ur statement 5 here; //to cfg EINT0 status LED pin as output pin using IODIR0 SFR

//add ur statement 6 here; //to enable External Interrupt 0 in VIC using VICIntEnable SFR

//add ur statement 7 here; //to enable External Interrupt 0 as Vector IRQ with priority 0 (highest)                                               			     //using VICVecCntl0 SFR

//add ur statement 8 here; //to write the External Interrupt 0’s ISR address into 					                    //VICVectAddr0

//add ur statement 9 here; // to enable EINT0 using EXTINT0 SFR

//add ur statement 10 here; //to cfg EINT0 as EDGE_TRIG using EXTMODE

//add ur statement 11 here; //to cfg EINT0 for RISING EDGE using EXTPOLAR

}

void eint0_isr(void) __irq

{

//add ur statement 12 here; //to toggle EINT0 status LED as confirmation of isr activity in action

// add ur statement 13 here; //to clear flag EINT0 status flag using EXTINT SFR

//add ur statement 14 here;  //to perform dummy write operation into VICVectAddr

}

- Write an ECP to allow External Interrupt 0 to be served ONLY 5 times ONLY for Falling Edge.

- Write an ECP to demonstrate priority of External Interrupts 0 & 1 for Falling Edge using debug scripting.

- Complete the Keil IDE’s debug script given below for show  External Interrupt 0 & 1 being fired at the same time and LPC2129 uC is responding to service the two interrupts being served in a prioritized manner.

/* external_interrupts_debug.ini */

___________   ___________ "TOGGLE_EINT_PINS" , "ToggleEINTPins(__,__)"

_______ void ToggleEINTPins(unsigned pin1, unsigned pin2, unsigned pin3,  unsigned pin4)

{

______^=((1<<pin1)|(1<<pin2)|(1<<pin3)|(1<<pin4));

}

- Write an ALP to demonstrate External Interrupt 0 for Falling Edge.

- ECP to demonstrate priority of External Interrupts 0 & 1 for Rising Edge.

/* external_interrupts_priority_test.c */

#include <lpc21xx.h>

#include "pin_function_defines.h"

#include "defines.h"

#define EINT0_LED 10

#define EINT1_LED 11

void delay_ms(unsigned dlyMS);

void eint0_isr(void) __irq;

void eint1_isr(void) __irq;

void Enable_EINT0(void);

void Enable_EINT1(void);

unsigned int count;

main()

{

Enable_EINT0();

Enable_EINT1();

while(1)  {  count++;  }

}

void delay_ms(unsigned dlyMS)

{

dlyMS*=12000;

while(dlyMS--);

}

| void eint0_isr(void) __irq { CPLBIT(IOPIN0,EINT0_LED);//isr activity //statement 1 to clr EINT0 status flag //statement 2 dummy write into //VICVectAddr delay_ms(100); } | void eint1_isr(void) __irq { CPLBIT(IOPIN0,EINT1_LED);//isr activity //statement 3 to clr EINT1 status flag //statement 4 to dummy write into //VICVectAddr delay_ms(100); } |
| --- | --- |
| void Enable_EINT0(void) { CFGPIN(PINSEL0,1,FUNC4); SETBIT(IODIR0,EINT0_LED); SSETBIT(VICIntEnable,14); //statements 5 to set priority for EINT0 as 0 ; //statement 6 to enable EINT0 ; //statement 8 cfg EINT0 as Edge Triggered; SETBIT(EXTPOLAR,0); } | void Enable_EINT1(void) { CFGPIN(PINSEL0,3,FUNC4); SETBIT(IODIR0,EINT1_LED); SSETBIT(VICIntEnable,15); //statements 9 to set priority for EINT0 as 1 ; //statement 10 to enable EINT0; //statement 8 cfg EINT1 as Edge Triggered; SETBIT(EXTPOLAR,1); } |