I2C Objective Questions Set 2

- Prior to any transaction on the bus, a _________condition needs to be issued on the bus.

- START

- STOP

- Repeated START

- None of the above

- The_________ condition acts as a signal to all connected IC's that something is about to be transmitted on the bus. As a result, all connected chips will listen to the bus.

- Master taking acknowledgement from slave

- Master giving acknowledgement to slave

- Repeated START

- None of the above

- After an I2C frame transmission/reception has been completed, a __________ condition is sent.

- Repeated START

- STOP

- START

- None of the above

- _________ condition is the signal for all devices on the bus that the bus is available again (idle). If a chip was accessed and has received data during the last transaction, it will now process this information (if not already processed during the reception of the message).

- Master issuing not of acknowledgment onto bus

- Master taking acknowledgement from slave

- Master giving acknowledgement to slave

- None of the above

- A single message can contain multiple ________ conditions. The  so-called "repeated _________" is used in I2C Communication.

- Start, Stop

- Start, Start

- Stop, Start

- Stop, Stop

- The first byte after a ________ condition will identify the slave on the bus (address) and will select _________ mode of operation.

- STOP, either READ/WRITE

- Repeated START ,both READ & WRITE

- START, either READ/WRITE

- START ,both READ & WRITE

- A _________ condition ALWAYS denotes the ________of a transmission. Even if it is issued in the middle of a transaction or in the middle of a byte. It is "good behavior" for a chip that, in this case, it disregards the information sent and resumes the "listening state", waiting for a new start condition.

- Start, BEGINNING

- Stop, END

- Start, END

- Stop, BEGINNING

- Given the waveform below, choice the appropriate option that best describes the waveform.

- Transmitting a Bit in a Byte to a Slave Device

- Receiving a Bit of Byte From a Slave Device

- Either A or B

- Both A and B

- Given the waveform below, choice the appropriate option that best describes the waveform.

- Transmitting a Bit in a Byte to a Slave Device

- Receiving a Bit of Byte From a Slave Device

- Transmitting a Byte to a Slave Device

- Receiving a Byte From a Slave Device

- Using the waveform of question 9, again choice the appropriate option that best describes the waveform.

- Transmitting slave address with write option to a slave device

- Transmitting slave address with read option to a slave device

- Transmitting slave address with write/read option to a slave device

- Transmitting slave address with write & read option to a slave device