I2C Objective Questions Set 3

- The following  is true about  “No Acknowledge Condition” in I2C protocol  .

- The slave is not there (in case of an address)

- The slave missed a pulse and got out of sync with the SCL line of the master.

- The bus is "stuck". One of the lines could be held low permanently.

- All of the above choice are correct.

- If, after transmission of the 8th bit from the master to the slave the slave does not pull the SDA line low, then this is considered a ___________ condition.

- Not  Acknowledge

- No Acknowledge

- Master getting acknowledgement from a slave(ACK).

- None of the choices are correct.

- When No Acknowledgement condition arises due to any one of the 3 known cases the _________ should abort by attempting to send a _______ condition on the bus.

- Master,Stop

- Master,Start

- Slave,Start

- Slave,Stop

- With regard to Bit Transfer on I2C Bus, in normal data transfer , the ________ line changes state when the _________  is low.

- SDA,SCL

- SCL,SDA

- SDA,SDA

- SCL,SCL

- Once the _______ has been addressed and the  ________ has acknowledged this, a byte can be received from the ________ if the R/W bit in the address was set to READ (set to '1') in I2C .

- Master,Master,Slave

- Master,Slave,Slave

- Slave,slave,Master

- Slave,slave,slave

- Once the _______ has addressed a ________ & the  ________ has acknowledged this, a byte can be received by the ________ if the R/W bit in the address was set to READ (set to '1') in I2C .

- Master,Master,Slave,Master

- Master,Slave,Slave,Master

- Slave,slave,Master,Master

- Slave,slave,slave,master

- With respect to the I2C Bus Event “Receiving a Byte From a Slave Device “,the correct order/sequence of the 3 statements(a,b,c) given below would be

- The line will then go high if it wants to transmit a '1' or, if the slave wants to send a '0', remain low.

- Prior to sending the 8 clock pulses needed to clock in a byte on the SCL line, the master releases the SDA line.

- The slave will now take control of this line.

- b,c,a

- c,a,b

- a,b,c

- b,a,c

- The I2C bus drivers are "open drain", meaning that they can pull the corresponding signal line ______, but cannot drive it ________.

- high, high

- low, high

- high, low

- low, low

- The maximum number of slaves that can be connected to an I2C bus depends upon

- Slave addressing method (7 bit / 10-bit)

- The total bus capacitance

- Both A & B

- None of the choices are correct

- Higher values of pull up resistors used  for I2C Bus imply _______  pull up, which ________ rise time(tr).Where as lower values of pull up resistors imply _______ pull up,which _________rise time(tr).

tr is __________ proportional to RC,where R is pull up resistor’s value, & C is total bus capacitance.

- Weak, increases, strong, decreases, directly

- Strong, increases, weak, decreases, directly

- Weak, decreases, strong, increases, inversely

- Strong, decreases, weak, increases ,inversely