LPC2129 ON CHIP I2C

- How many I2C on chip peripherals are available on LPC2129?

- 1		B) 2		C) 3		D) 4

- In LPC2129, _______ & _______ pins are used for on-chip I2C functionality.

- In LPC2129, ________ number of SFRS are available under I2C register map.

- In LPC2129, PINSEL1 is used for selecting the I2C functionality. (TRUE/FALSE)

- Which functionality of P0.2 has SCL functionality?

- FUNC3	B) FUNC1	C) FUNC2	D) FUNC4

- Which functionality of P0.3 has SDA functionality?

A) FUNC1	B) FUNC3	C) FUNC2	D) FUNC4

- What is the maximum speed of I2C communication on LPC2129?

- 100 Kbps		B) 400 Kbps		C) 1 Mbps		D) 2 Mbps

- SDA pin is I2C data input/output pin for I2C communication. (TRUE/FALSE)

- SCL pin is I2C data output pin for I2C communication. (TRUE/FALSE)

- Which statement is used for configure the P0.2 pin for SCL function?

- CFGPIN(PINSEL0,2,FUN1)	B) CFGPIN(PINSEL0,2,FUN2)

C)  CFGPIN(PINSEL0,2,FUN3)	D) CFGPIN(PINSEL0,2,FUN4)

- Which statement is used for configure the P0.3 pin for SDA function?

- CFGPIN(PINSEL0,3,FUN1)	B) CFGPIN(PINSEL0,3,FUN2)

C)  CFGPIN(PINSEL0,3,FUN3)	D) CFGPIN(PINSEL0,3,FUN4)

- LPC2129 provides internally a ______ oriented I2C interface between ARM processor and I2C interface.

A) BIT		B) NIBBLE	C) BYTE 	D) None of the mentioned

- LPC2129 provides a ______ oriented I2C interface with external I2C bus.

A) BIT		B) NIBBLE	C) BYTE 	D) None of the mentioned

- What is the size of the I2C data buffer in LPC2129?

- 8 bits	B) 16 bits	C) 32 bits	D) 64 bits

- Which registers are used to control the I2C interface on LPC2129?

- PINSEL0 and PINSEL1

- I2CONSET and I2CONCLR

- VICVectAddr0 and VICVectAddr1

- PLL0CON and PLL0FEED

- Which registers are used to set the data rate of I2C interface on LPC2129?

- PINSEL0 and PINSEL1

- I2CONSET and I2CONCLR

- I2SCLH and I2SCLL

- PLL0CON and PLL0FEED

- What is the size of I2CONSET register in LPC2129?

A) 16 bits	B) 32 bits	C) 8 bits	D) None of the mentioned

- What is the size of I2CONCLR register in LPC2129?

A) 16 bits	B) 8 bits	C) 32 bits	D) None of the mentioned

- What is the size of I2SCLH register in LPC2129?

A) 16 bits	B) 32 bits	C) 8 bits	D) None of the mentioned

- What is the size of I2SCLL register in LPC2129?

A) 32 bits	B) 16 bits	C) 8 bits	D) None of the mentioned

- Which bit in I2CONSET register is used to generate a start condition?

- Bit1	B) Bit2		C) Bit3		D) None of the mentioned

- Which bit in I2CONSET register is used to generate a stop condition?

- Bit1	B) Bit2		C) Bit3		D) None of the mentioned

- When _______ bit is 1 in master mode, a STOP condition is transmitted on the I2C bus.

- STA	B) STO	C) SI		D) None of the mentioned

- When _______ bit is 1 in master mode, a START condition is transmitted on the I2C bus.

- STA	B) STO	C) SI		D) None of the mentioned

- In LPC2129, SI is the I2C _________ flag.

A) Status 	B) Interrupt	C) Sync	D) All of the mentioned

- Which bit in I2CONSET register is used to enable the I2C interface enable?

- Bit3	B) Bit4		C) Bit5		D) Bit6

- STA bit can be cleared by writing 0 to the STAC bit in the I2CONCLR register. (TRUE/FALSE)

- STA bit can be cleared by writing 0 to the STA bit in the I2CONSET register. (TRUE/FALSE)

- STA bit can be cleared by writing 1 to the STAC bit in the I2CONCLR register. (TRUE/FALSE)

- The STO flag is cleared by hardware automatically. (TRUE/FALSE)

- In LPC2129, SI bit is set when one of the _______ possible I2C states is entered.

- 4		B) 20		C) 25		D) None of the above

- I2C SCL Duty cycle registers are ______ & ______.

- In LPC2129, Bit frequency is equal to pclk/____________.