LPC2129 ON CHIP SPI

- How many SPI on chip peripherals are available on LPC2129?

- 1		B) 2		C) 3		D) 4

- Which of the following are used for SPI0 communication In LPC2129?

- P0.17, P0.18, P0.19, P0.20

- P1.17, P1.18, P1.19, P1.20

- P0.4, P0.5, P0.6, P0.7

- None of the mentioned

- Which of the following are used for SPI1 communication In LPC2129?

- P0.4, P0.5, P0.6, P0.7

- P1.17, P1.18, P1.19, P1.20

- P0.17, P0.18, P0.19, P0.20

- None of the mentioned

- In LPC2129, ________ number of SFRS are available under SPI register map.

- List out the SPI registers in LPC2129?

- In LPC2129, PINSEL0 is used for selecting the SPI functionality. (TRUE/FALSE)

- Which functionality of P0.4 has SCK0 functionality?

- FUNC3	B) FUNC1	C) FUNC2	D) FUNC4

- Which functionality of P0.5 has MISO0 functionality?

A) FUNC1	B) FUNC3	C) FUNC2	D) FUNC4

- Which functionality of P0.6 has MOSI0 functionality?

- FUNC1	B) FUNC2	C) FUNC3	D) FUNC4

- MISO0/MISO1 pins are data input to SPI master as well as SPI slave. (TRUE/FALSE)

- MISO0/MISO1 pins are data input to SPI master or data output from SPI slave. (TRUE/FALSE)

- MOSI0/MOSI1 pins are data output from SPI master or data input to SPI slave. (TRUE/FALSE)

- MOSI0/MOSI1 pins are data output from SPI master as well as SPI slave. (TRUE/FALSE)

- SSEL0 used to select the SPI interface as a slave. (TRUE/FALSE)

- Which statement is used for configure the P0.4 pin for SCK0 function?

- CFGPIN(PINSEL0,4,FUN1)	B) CFGPIN(PINSEL0,4,FUN2)

C)  CFGPIN(PINSEL0,4,FUN3)	D) CFGPIN(PINSEL0,4,FUN4)

- Which statement is used for configure the P0.5 pin for MISO0 function?

- CFGPIN(PINSEL0,5,FUN1)	B) CFGPIN(PINSEL0,5,FUN2)

C)  CFGPIN(PINSEL0,5,FUN3)	D) CFGPIN(PINSEL0,5,FUN4)

- Which statement is used for configure the P0.6 pin for MOSI0 function?

A)  CFGPIN(PINSEL0,6,FUN1)	B) CFGPIN(PINSEL0,6,FUN2)

C)  CFGPIN(PINSEL0,6,FUN3)	D) CFGPIN(PINSEL0,6,FUN4)

- What is the size of the SPI data register in LPC2129?

- 8 bits	B) 16 bits	C) 32 bits	D) 64 bits

- Which registers are used to control the SPI interface on LPC2129?

- PINSEL0 and PINSEL1

- S0SPCR and S1SPCR

- VICVectAddr0 and VICVectAddr1

- PLL0CON and PLL0FEED

- Which registers are used to set the data rate of SPI interface on LPC2129?

- PINSEL0 and PINSEL1

- S0SPCCR and S1SPCCR

- I2SCLH and I2SCLL

- I2CONSET and I2CONCLR

- What is the size of SPCR register in LPC2129?

A) 16 bits	B) 32 bits	C) 8 bits	D) None of the mentioned

- What is the size of SPSR register in LPC2129?

A) 16 bits	B) 8 bits	C) 32 bits	D) None of the mentioned

- What is the size of SPDR register in LPC2129?

A) 16 bits	B) 32 bits	C) 8 bits	D) None of the mentioned

- What is the size of SPCCR register in LPC2129?

A) 32 bits	B) 16 bits	C) 8 bits	D) None of the mentioned

- Which mode of operation of SPI is supported by LPC2129?

- Master mode only

- Slave mode only

- Both master and slave modes

- None of the mentioned

- In SPCR register, _____ & _____ bits are representing CPOL & CPHA.

- In SPCR register, ______ bit is used to select the mode of operation.

- SPDR is a bi-directional data register. (TRUE/FALSE)

- The value which is loaded into the SPCCR register must be always be an even value. (TRUE/FALSE)

- What does SPIF stands for in LPC2129?

- Serial peripheral interface flag

- Serial peripheral interrupt flag

- SPI transfer complete flag

- None of the mentioned