CAN OBJECTIVE QUESTIONS SET 2

- A CAN network has the following underlying architecture:

a) Master-slave system and line topology

b) Master-slave system and ring topology

c) Multi-master system and line topology

d) Multi-master system and ring topology

- In a CAN network, all CAN nodes are independent of one another.

a) True  	b) False

- Which of one these message types are described in the CAN protocol?

a)  data frame	    			b)  abitration frame

c)   sporadic frame   			d) Event triggered frame

- Which of these fields are not parts of the data frame?

a)  Data Field				b)  Arbitration Field

c)   Synchronization Break Field	d) CRC Field

- Which of the below in a data frame field serves to differentiate between a data frame and a remote frame?

a)  Identifier Extension Bit (IDE)		b)  Data Length Code Bits (DLC)

c)  Remote Transmission Request Bit (RTR)		d) None of the above

- Which of the below in a data or remote frame does the Cyclic Redundancy Check protect?

a)  Identifier, DLC		b)  IFS/ITM

c)  Acknowledgement Field	d) End Of Frame (EOF)

- Let us assume that all receivers acknowledge transmission of a data frame negatively. Which of the following errors does the sender detect?

a)  Bit Monitoring Error		b)  Stuffing Error

c)  Format Error			d) Acknowledgement Error

- The so-called data frame is used for data transmission in a CAN network. What is the maximum number of data bits that can be transmitted with a data frame?

a)  8 bits	b)  16 bits	c)  32 bits	d) 64 bits

- Is it true that the identifier of a data or remote frame addresses a specific CAN node?

a)  Yes		b)  No

- The CAN protocol specifies that receivers acknowledge each data and remote frame. Is it true that the acknowledgement is made after transmission of the data or remote frame?

a) Yes 		b) No

- During transmission of a data or remote frame, the sender inserts a complementary bit after a certain number of homogeneous bits for synchronization purposes. What is this number?

a) 3 		b) 4		c) 5		d) 6

- By which method do the CAN nodes access the CAN bus?

a)  Carrier Sense Multiple Access (CSMA)

b)  Token Ring

c)  Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA)

d) Round Robin