CAN OBJECTIVE QUESTIONS SET 3

- The identifier (ID) represents the priority of the data or remote frame. What is the relationship between the identifier and the priority?

a)  None of the mentioned

b)  The higher the ID the lower the priority

c)  The higher the ID the higher the priority

d)  The lower the ID the higher the priority

- Is it true that the nodes in a CAN network have a priority?

a) Yes 	b) No

- What would be the sequence of data frames transmitted on the CAN bus, if they are all ready to transmit at the same time? Give answer using numbers 1,2,3,4 beside frames.

a)  Data Frame with ID=0x476  : ______

b)  Data Frame with ID=0x45   : ______

c)  Data Frame with ID=0x135 : ______

d)  Data Frame with ID=0x97   : ______

- Is it true that each CAN node is granted immediate access to the CAN bus if necessary?

a)  Yes		b)  No

- A CAN node wishes to access the CAN bus during transmission of another data frame. When is the earliest point at which the CAN node can access the CAN bus?

a)  Immediately after the transmission of the data frame

b)  After the ITM (Intermission)/IFS has expired

c)  After the acknowledgement

d)  During transmission of the EOF (End Of Frame)

- Is it true that with increasing bus load there is an increased probability that transmission of lower priority CAN messages will be delayed?

a)  Yes		b)  No

- Which of the following is a precondition that must be satisfied for a workable bitwise arbitration?

a) Positive Acknowledgement 		b)  Synchronism of the Senders

c)  Even-numbered Identifiers		d) None of the above

- Which of the following properties would you use to describe CAN bus access?

a)  Predictable    			b)  Random

c)  Time-controlled    			d)  Collision-free

- According to the CAN protocol, can the data frame with ID=0x34 ever be placed on the bus at the same time by two different CAN nodes?

a)  Yes 		b)  No

- Which mechanisms are part of the logical data protection in a CAN network?

a)  All Mentioned   b)  Error Detection    c)  Error Handling    d) Error Tracking

- The CAN protocol describes five error detection mechanisms. Some are only performed by the sender, and others are only handled as a receiver. Which error detection mechanisms are executed by which CAN node in the transmission of a data or remote frame?

a)  Bit Monitoring --  ______.

b)  Bit Stuffing Check -- ____________.

c)  Form Check -- ____________.

d) ACK Check -- _____________.

e)  Cyclic Redundancy Check -- ____________.

- After detecting a protocol error, a CAN controller transmits six recessive bits. In which state is the CAN controller?

a)  Active

b)  Error Passive

c)  Error Active

d) Bus Off