CAN OBJECTIVE QUESTIONS SET 5

- The CAN protocol describes five error detection mechanisms. Match the error detection mechanisms to their functions.

a)  Bit Stuffing Check                       1 )Checks the bit stuffing rule

b)  Form Check                                 2) Checks cyclic redundancy

c)  ACK Check                                  3) Checks the sent bits

d) Cyclic Redundancy Check          4) Checks the ACK slot

e) Bit Monitoring                              5) Check the format of a CAN message

- In which cases is sending of six homogeneous bits allowed?

a)  In transmitting the identifier

b)  In error signaling

c)  In transmitting the EOF

d)  In transmitting the SOF

- Which preconditions must be fulfilled for a CAN node to switch to the error passive state?

a)  CAN node is in the error active state

b)  CAN node is in the bus off state

c)  Transmit Error Counter (REC) is greater than 255

d) Receive Error Counter (REC) is greater than 255

- An error-detecting CAN node transmits an error flag to assure network-wide data consistency. Afterwards, all other nodes in the CAN network similarly transmit an error flag. How many bits does this secondary error flag have?

a)  2

b)  4

c)  6

d) 8

- Is it true that an error-detecting CAN node must transmit an error delimiter after the error flag?

a)  Yes		b)  No

- As a sender, a CAN node in the “Error passive” state cannot inform other nodes in the CAN network of a detected error. Is this true?

a)  Yes		b)  No

- Which of the following data protection mechanisms are not physical data protection mechanisms in a CAN network?

a)  Differential Signal Transmission

b)  Bit Monitoring

c)  Bit Stuffing

d)  Sheilding

- During the “Ack Slot Bit”,the transmitting node sends out ______________ bit.Any node that has received an error-free frame acknowledges the correct reception of the frame by sending back / broadcasting a ___________ bit.

a)  recessive,dominant

b) dominant, recessive

c) dominant,dominant

d) recessive,recessive

- The total number of fields that can be grouped in the data frame are _________

a)  4		b)  5		c)   6		 d) 7

- All nodes on a given CAN bus must have the same nominal bit rate (NBR).

a)  True

b) False

- NBR is defined in the CAN specification as the number of bits per second transmitted by an ideal transmitter with no resynchronisation.

a)  True

b) False

- CAN bit time segments are _____ in number.

a)  1		b) 2		a)  3		c) 4