CAN OBJECTIVE QUESTIONS SET 5

- The fixed segment in CAN bit timing is called ____________.

a)  SyncSeg		b) PropSeg		a) PS1		c) PS2

- The minimal periods of time in CAN protocol/bus for a bit time is called ____________.

a)  Nominal Bit time(NBT)			b) Nominal Bit Rate(NBR)

c) Synchronisation Jump Width (SJW)	d) Time Quanta(TQ)

- Which of the below will be involved during Message filtering for data or remote frame ____________.

a)  SOF	b) RTR	c) Identifier	d) None of the Mentioned

- The Interframe Space(IFS) is composed of atleast   ____ or more  __________ bits.

a)  3,dominant	b) 3,recessive		c) 1,recessive		d) 2,recessive

- Bit Stuffing Rule is applicable between SOF to CRC’s LSB inclusive .

a)  True			b) False

- The EOF Field is composed of  _____ ___________ bits.

a)  7,dominant	b) 7,recessive		c) 6,recessive		d) 5,recessive

- Upon Reset any CAN node is said to be in _____________ state.

a)  Error-Active	b) Error-Passive	c) Bus-Off	d) Unknown

- Any CAN node switch from Error-Active state to _____________ state if REC/TEC >127.

a)  Error-Active	b) Error-Passive	c) Bus-Off	d) Unknown

- Any CAN node switch from Error-Passive state to _____________ state if REC/TEC <127.

a)  Error-Active	b) Error-Passive	c) Bus-Off	d) Unknown

- Any CAN node switches from Error-Passive state to ______________ state if TEC >255.

a)  Error-Active	b) Error-Passive	c) Bus-Off  	d) Unknown

- Any CAN node switches from Bus-Off state to ______________ state if it is RESET .

a)  Error-Active	b) Error-Passive	c) Bus-Off	d) Unknown