CAN OBJECTIVE QUESTIONS SET 6

- Any CAN node switches from Bus-Off state to Active-Error state if______________.

a)  it is RESET		b)  128 occurrences of 11 consecutive “recessive” bits on bus

c)  a or b options		d) a & b options

- If a CAN node is alone on the bus and tries to transmit then  ____ is/are true.

a)  The node will win the arbitration and proceed with the message transmission.

b)  During the ACK slot,no node will send a dominant bit so the transmitter will sense an ACK  	error.

c)  a & b options

d) no options

- Which among the following is a CAN transceiver ______________.

a)  AT89S52	b)  MCP2551		c)  P89V51RD2	d) MCP2515

- MCP2515 CAN controller implements ______ at 1Mb/s .

a)  CAN2.0A	b)  CAN2.0B		c)  CAN FD	d) all of the mentioned

- MCP2515 CAN controller support  SPI modes ______________ at ______ MHz max.

a)  0,3,10		b)  1,2,10		c)  0,3,5	d) 1,2,5

- The Bit Timing Configuration registers in MCP2515 CAN controller are _______.

a)  CNF1		b)  CNF2		c)  a & b	d) c & more

- When using MCP2515 CAN controller(TQ= 2 . BRP. TOSC),assuming that 125kHz CAN baud rate with Fosc=20MHz is desired. TOSC = 50 ns, choose BRP =5, then TQ = 500 ns. To obtain 125 kHz, the bit time must be  ____  TQ.

a)  5		b)  10		c)  16		d) 18

- The MCP2515 has _____modes of operation. And in ________ mode ONLY which transmit / receive frames.

a)  5,Configuration	b)  5,Normal		c)  5,Loop back	d) 3,Normal

- When using MCP2515 CAN controller's SPI Interface  _______  instruction,provides a means   for setting or clearing individual bits in specific status and control registers.

a)  Bit Modify		b)  Reset		c)  Read	d) Write

- When using MCP2515 CAN controller's SPI Interface  _______  waveform is used for setting or clearing individual bits in specific status and control registers.

a)

b)

c)

d) None of the above.

- When using MCP2515 CAN controller's SPI Interface  _______  waveform given in options of question 69 is used for  “Byte Write Instruction”.

- Given below are two CAN nodes. Identify & Complete the connectivity  between CAN controller & CAN Transceiver of each node and also between the two CAN nodes ,by drawing directed lines.