OBJECTIVE & SHORT ANSWERS QUESTIONS  ON CAN BUS PROTOCOL

- CAN bus is ______wire ____________ pair, ____________bus,and support ONLY ________________ serial communication.

- Map the following LHS with RHS .

a) DATA FRAME	i)  “Hello everyone,can somebody please produce the data  labelled  X ?”

b) REMOTE FRAME         	ii)  “OH DEAR, LET’S TRY AGAIN”

c) ERROR FRAME             	iii) “I’m a very busy node , could you please wait for a   moment”

d) OVERLOAD FRAME	 iv) “Hello everyone,here’s some data labelled X,hope U like it”

- There are  _______ different message types( or “frames”) on a CAN bus.They are ______________, _______________, _______________,& ________________.

- The _____________ & ____________ frames violates the framing rules of a CAN message.

- The _____________ frame is transmitted when a node detects a fault and will cause all other nodes to detect a fault – so they will send __________ frame, too.

- Map the recommended maximum cable lengths with CAN bus speeds.

a) 40m	                 i)  10 kbits/s

b)  100m                   ii)  125 kbits/s

c) 200m                    iii) 500 kbits/s

d) 500m	                iv) 250 kbits/s

e) 6km		    v) 1Mbs/s

- The CAN protocol defines ______ different ways of detecting error. _______ of these works at the bit level, and the other  _______ at the message level.