ARM7 ADDRESSING MODES & INSTRUCTIONS

- _____________ is a tool used for creating new / modifying existing  & then saving, assembly/C sources files.

- _____________ is a tool used for converting assembly source code file(s) into machine understandable object codes file(s) ,which are binary files but not executable yet.

- _____________ is a tool used for combining the user object code file with predefined library object code files (if required )or combining 2 or more user object codes file with predefined library object code files if required to generate/produce a single executable hex file.

- _____________ is a tool used for downloading/flashing/burning the executable file’s information into the flash memory of the micro-controller for it to execute it as an application.

- _____________ is a tool used for testing the flashed application whether it is logically behaving as expected or not ,to decide whether corrections are required or not in the application.

- _____________ is a tool which mimics the behaviour of the actual system, in a virtual way, on a different host (mostly but not always a desktop pc/laptop) , to test/validate the application’s input , behaviour, & output without the availability of the actual hardware.

- An ___________ refers to the collection of tools such editor, assembler (native/cross) , compiler (native/cross) , preprocessor,linker,loader,debugger,simulator in one package for software developments.

- Assembly Language is usually considered not source portable language . (True/False)

- Assembly Language is not case sensitive programming language. (True/False)

- Assembly Language is machine specific programming language. (True/False)

- Assembly Language uses abbreviations & symbols to represent opcodes & operands of CPU instructions machine codes. (True/False)

- _____________ refers to manner/ways of how operands are mentioned in assembly language instructions.

- The ARM 7 Core supports _____ Addressing Modes.

- In _______________ addressing mode , the data (preceded by symbol ‘#’) is given in the instruction.

- In _______________ addressing mode, the data is given by a register. And is the mode used for ALU operations.

- In _______________ addressing mode, the address of the operand is given as a label/symbol in the instruction for the Assembler, valid for load operations ONLY . But actual would be interpreted as by assembler into register indirect addressing mode type at machine level.

- In ___________________ addressing mode , the address of the memory operand is given by the register.

- In _________________________________ addressing mode, the address of the memory operand is given by the register plus a numeric displacement/offset.

- In _________________________________ addressing mode, the address of the memory operand is given by the sum of two registers, where first register acts as base, and the second register acts as index/offset/displacement holding register .

- In __________________________________ addressing mode, the address of the memory operand is given by the sum of two registers, where first register acts as base, and the second register can be scaled by shifting left .

- The instruction format “MOV Rn,#8bit-number” is an example of ______________ addressing mode.

- The instruction format “MOV Rn, Rm” is an example of ______________ addressing mode.

- The instruction format “LDR Rn, Label” is an example of ______________ addressing mode, from assembler point of view but actual treated as register indirect addressing mode at machine level.

- The instruction format “LDR Rn,[Rm]” is an example of ___________________________________ addressing mode.

- The instruction format “LDR Rn,[Rm, #offset]” is an example of _____________________________ addressing mode.

- The instruction format “LDR Rd,[Rn, Rm]” is an example of __________________________________ addressing mode.

- The instruction format “LDR Rd,[Rn ,Rm, LSL #N]” is an example of __________________________________ addressing mode.

ARM7 ADDRESSING MODES & INSTRUCTIONS Cont’d

- Represent the operations given below with their appropriate ARM assembly instructions

Operations                                                                                          Instructions

- R0  Value of R1 + data value 25H		            :

- R0  Value of R1 + Value of R2                                   :

- R0 data from memory location “lablel”                 :

- R0 data from memory location pointed by R1      :

- R0 data from memory location pointed by

( R1 + 5). R1 remain unaltered                                     :

- R0  data from memory location pointed by

(R1 + R2). R1 remain unaltered                                   :

- R0  data from memory location pointed by

(R1 + R2 left shifted by 2). R1 remain unaltered      :

- Represent the operations given below with their appropriate ARM Addressing Modes

Operations                                                                                          Instructions

- R0  Value of R1 + data value 25H		            :

- R0  Value of R1 + Value of R2                                   :

- R0 data from memory location “label”                 :

- R0 data from memory location pointed by R1      :

- R0 data from memory location pointed by

(R1 +  5). R1 remain unaltered                                    :

- R0  data from memory location pointed by

(R1 + R2). R1 remain unaltered                                   :

- R0  data from memory location pointed by

(R1 + R2 left shifted by 2). R1 remain unaltered     :

- MAP the operations given below with their appropriate ARM assembly instructions Modes

Operations                                                                                          Instructions

- R0  Value of R1 + data value 25H		            :     LDR R0 , label

- R0  Value of R1 + Value of R2                                   :      ADD R0 ,R1 , #25H

- R0 data from memory location “lablel”                  :      LDR R0 , [R1 , R2 , LSL #2]

- R0 data from memory location pointed by R1      :     LDR R0, [ R1 , R2]

- R0 data from memory location pointed by R1

(R1 +  5 ). R1 remain unaltered                           :      LDR R0 , [ R1,#05H ]

- R0  data from memory location pointed by

(R1 + R2 ). R1 remain unaltered                          :      LDR R0 , [ R1 ]

- R0  data from memory location pointed by

(R1 + R2 left shifted by 2 ).

R1 remain unaltered                                           :      ADD R0 , R1 , R2

ARM7 ASSEMBLY EXERCISES

- For the code given below

MOV R0,#0x12

MOV R1,R0,LSR #4

R0 , R1  would have the values ________ & ________ respectively.

- For the code given below

MOV R0,#0x12

MOV R1,R0,LSL #28

MOV R1,R1,LSR #28

R0 , R1  would have the values ________ & ________ respectively.

- For the code given below

MOV R0,#0x12

MOV R1,R0,LSL #28

MOV R1,R1,LSR #28

R0 , R1  would have the values ________ & ________ respectively.

- For the code given below

LDR R0,=0x1234

MOV R1,R0,LSR #8

MOV R2,R0,LSL #24

MOV R2,R2,LSR #24

R0 , R1 , & R3 would have the values ___________,  ________  &  ________ respectively.

- For the code given below

LDR R0,=0x1234

MOV R1,R0,LSR #12

MOV R2,R0,LSL #20

MOV R2,R2,LSR #20

MOV R3,R0,LSR #4

MOV R3,R3,LSL #28

MOV R3,R3,LSR #28

MOV R4,R0,LSL #28

MOV R4,R4,LSR #28

R0 , R1 , R2,R3 would have the values ___________,  ________  , _________&  ________ respectively.

- For the code given below

LDR R0,=0x12345678

MOV R1,R0,LSR #28

MOV R2,R0,LSR #24

MOV R2,R2,LSL #28

MOV R2,R2,LSL #28

MOV R3,R0,LSL #8

MOV R3,R3,LSR #28

MOV R4,R0,LSL #12

MOV R4,R4,LSL #28

R0 , R1 , R2, R3, R4 would have the values _________ ,  ________  , _________, ________ & ________ respectively.

- For the code given below

LDR R0,=0x12345678

MOV R5,R0,LSL #16

MOV R5,R5,LSR #28

MOV R6,R0,LSL #20

MOV R6,R6,LSR #28

MOV R7,R0,LSL #24

MOV R7,R7,LSR #28

MOV R8,R0,LSL #28

MOV R8,R8,LSR #28

R0 , R5 , R6, R7, R8 would have the values _________ ,  ________  , _________, ________ & ________ respectively.

- For the code given below

LDR R0,=0x12345678

MOV R1,R0,LSR #28

MOV R2,R0,LSR #24

MOV R2,R2,LSL #28

MOV R2,R2,LSL #28

MOV R3,R0,LSL #8

MOV R3,R3,LSR #28

MOV R4,R0,LSL #12

MOV R4,R4,LSL #28

MOV R5,R0,LSL #16

MOV R5,R5,LSR #28

MOV R6,R0,LSL #20

MOV R6,R6,LSR #28

MOV R7,R0,LSL #24

MOV R7,R7,LSR #28

MOV R8,R0,LSL #28

MOV R8,R8,LSR #28

R0 , R1 , R2, R3, R4, R5, R6,R7,R8 would have the values ____________ , ____ ,  ____,____  , _____,____, ____,____ , ____ &____ respectively.