ARM7 ASSEMBLY DATA TRANSFER EXERCISES

- For the code given below, R0 , R1  would finally have the values ________ & ________ respectively.

MOV R0,#0x12

MOV R1,R0,LSR #4

- For the code given below , R0 , R1  would finally have the values ________ & ________ respectively.

MOV R0,#0x12

MOV R1,R0,LSL #28

MOV R1,R1,LSR #28

- For the code given below R0 , R1 , & R2  would finally have the  values ________ , ________ & ________ respectively.

MOV R0,#0x12

MOV R1,R0,LSR #4

MOV R2,R0,LSR #28

MOV R2,R2,LSL #28

- For the code given below , R0 , R1 , & R2 would finally have the values ___________,  ________  &  ________ respectively.

LDR R0,=0x1234

MOV R1,R0,LSR #8

MOV R2,R0,LSL #24

MOV R2,R2,LSR #24

- For the code given below , R0 , R1 , R2,R3 & R4would finally have the values ___________,  ________  , _________ , ________&  ________ respectively.

LDR R0,=0x1234

MOV R1,R0,LSR #12

MOV R2,R0,LSL #20

MOV R2,R2,LSR #20

MOV R3,R0,LSR #4

MOV R3,R3,LSL #28

MOV R3,R3,LSR #28

MOV R4,R0,LSL #28

MOV R4,R4,LSR #28

- For the code given below,R0 , R1 , R2, R3, R4 would finally have the values _________ ,  ________  , _________, ________ & ________ respectively.

LDR R0,=0x12345678

MOV R1,R0,LSR #28

MOV R2,R0,LSR #24

MOV R2,R2,LSL #28

MOV R2,R2,LSL #28

MOV R3,R0,LSL #8

MOV R3,R3,LSR #28

MOV R4,R0,LSL #12

MOV R4,R4,LSL #28

- For the code given below

LDR R0,=0x12345678

MOV R5,R0,LSL #16

MOV R5,R5,LSR #28

MOV R6,R0,LSL #20

MOV R6,R6,LSR #28

MOV R7,R0,LSL #24

MOV R7,R7,LSR #28

MOV R8,R0,LSL #28

MOV R8,R8,LSR #28

R0 , R5 , R6, R7, R8 would finally have the values _________ ,  ________  , _________, ________ & ________ respectively.

- For the code given below

LDR R0,=0x12345678

MOV R1,R0,LSR #28

MOV R2,R0,LSR #24

MOV R2,R2,LSL #28

MOV R2,R2,LSL #28

MOV R3,R0,LSL #8

MOV R3,R3,LSR #28

MOV R4,R0,LSL #12

MOV R4,R4,LSL #28

MOV R5,R0,LSL #16

MOV R5,R5,LSR #28

MOV R6,R0,LSL #20

MOV R6,R6,LSR #28

MOV R7,R0,LSL #24

MOV R7,R7,LSR #28

MOV R8,R0,LSL #28

MOV R8,R8,LSR #28

R0 , R1 , R2, R3, R4, R5, R6,R7,R8 would finally have the values ____________ , ____ ,  ____,____  , _____,____, ____,____ , ____ &____ respectively.

- For the code given below

MOV R0,#0x12

LDR   R1, = 0x4000000

STRB R0, [R1]

LDRB R2,[R1]

R0 , R1 , R2, &RAM LOCATION 0x40000000 would finally have the values ____________ , ____________ ,  ______________&____________respectively.

- For the code given below

LDR  R0,=0x1234

LDR   R1, = 0x4000000

STRH R0, [R1]

LDRH R2,[R1]

R0 , R1 , R2, & RAM LOCATIONS 0x40000000,0x40000001 would finally have the values ____________ , ____________ ,  ______________,_____________ & ____________respectively.

- For the code given below

LDR  R0,=0x12345678

LDR   R1, = 0x40000000

STR R0, [R1]

LDR R2,[R1]

R0 , R1 , R2, & RAM LOCATIONS 0x40000000,0x40000001,0x40000002,0x40000003 would finally have the values ____________ , ____________ ,  ______________, _____________, _____________, ______________&____________respectively.

- For the code given below

LDR  R0, = 0x12345678

LDR  R1, = 0x78563412

MOV R2,R0

MOV R0,R1

MOV R1,R2

R0 , R1 , R2, would  finally have the values ___________________ , _________________   &  ______________ respectively. Hence the registers _____ &_____ would be swapped.