- Add the appropriate instruction to replace ,where ever, the “ADD UR INSTRUCTION HERE” is available in the code given below.

;FILE : BIT_OPERATIONS_MACROS_SUBROUTINES_SINGLE.S

AREA MYDATA,DATA,READWRITE

RAM_LOC SPACE 4

AREA RESET,CODE,READONLY

ARM

ENTRY

;macros should be defined before macro use

;macros definitions begins here

MACRO

;ADD UR INSTRUCTION HERE  ; MACRO NAME WITH  REQUIRED ARGUMENTS

LDR R0,= $WORDVAR               ;HOLD MEM ADDR

MOV R1,#$BITPOS                    ;HOLD BIT POSITION TO SET

;ADD UR INSTRUCTION HERE  ; CALL THE SUBROUTINE

MEND

MACRO

;ADD UR INSTRUCTION HERE  ; MACRO NAME WITH REQUIRED ARGUMENTS

LDR R0,= $WORDVAR              ; R0 HOLDS MEM ADDR

MOV R1,#$BITPOS                    ;HOLD BIT POSITION TO CLEAR

;ADD UR INSTRUCTION HERE ;CALL THE SUBROUTINE

MEND

MACRO

;ADD UR INSTRUCTION HERE  ; MACRO NAME WITH REQUIRED ARGUMENTS

LDR R0,= $WORDVAR ;HOLD MEM ADDR

MOV R1,#$BITPOS   ;HOLD BIT POSITION TO WRITE

;ADD UR INSTRUCTION HERE ;CALL THE SUBROUTINE

MEND

MACRO   ; Macro definition WRITING BIT INTO ANY MEMORY

;ADD UR INSTRUCTION HERE  ; MACRO NAME WITH REQUIRED ARGUMENTS

LDR R0,= $WORDVAR ;HOLD MEM ADDR

MOV R1,#$BITPOS   ;HOLD BIT POSITION TO WRITE

MOV R2,#$BITLEVEL ;HOLD BIT LEVEL TO WRITE

;ADD UR INSTRUCTION HERE ;CALL THE SUBROUTINE

MEND

MACRO   ; Macro definition READING BIT FROM ANY MEMORY

;ADD UR INSTRUCTION HERE  ; MACRO NAME WITH REQUIRED ARGUMENTS

LDR R0,= $WORDVAR ;HOLD MEM ADDR

MOV R1,#$BITPOS   ;HOLD BIT POSITION TO WRITE

;ADD UR INSTRUCTION HERE ;CALL THE SUBROUTINE

MEND

;macros definitions end here

START   ;user main program begins execution here

SETBIT RAM_LOC,31

READBIT RAM_LOC,31

CLRBIT RAM_LOC,31

READBIT RAM_LOC,31

CPLBIT RAM_LOC,31

READBIT RAM_LOC,31

WRITEBIT RAM_LOC,31,1

READBIT RAM_LOC,31

WRITEBIT RAM_LOC,31,0

READBIT RAM_LOC,31

HERE B HERE ;user main program ends execution here

;SUB-ROUTINES FOR SETTING BIT IN BITPOS IN MEMORY

SR_SETBIT

LDR R3,[R0]                    ;GET WORD FROM MEMORY POINTED BY R0 INTO R3

MOV R4,#1                     ;

;ADD UR INSTRUCTION HERE      ;R4=R4<<R1

;ADD UR INSTRUCTION HERE      ;R3|=R4

STR R3,[R0]                    ;UPDATE WORD BACK FROM R3 INTO MEMORY POINTED BY R0

MOV PC,LR                    ;EXIT SUB-ROUTINE SR_SETBIT

;SUB-ROUTINES FOR SETTING BIT IN BITPOS IN MEMORY

SR_CLRBIT

LDR R3,[R0]      ;GET WORD FROM MEMORY INTO R3

MOV R4,#1        ;

;ADD UR INSTRUCTION HERE      ;R4=R4<<R1

;ADD UR INSTRUCTION HERE      ;R3&=~R4

STR R3,[R0] ;UPDATE WORD BACK FROM R3 INTO MEMORY

MOV PC,LR  ;EXIT SUB-ROUTINE SR_CLRBIT

;SUB-ROUTINES FOR SETTING BIT IN BITPOS IN MEMORY

SR_CPLBIT

LDR R3,[R0]      ;GET WORD FROM MEMORY INTO R3

MOV R4,#1        ;

;ADD UR INSTRUCTION HERE      ;R4=R4<<R1

;ADD UR INSTRUCTION HERE      ;R3^=R4

STR R3,[R0]      ;UPDATE WORD BACK FROM R3 INTO MEMORY

MOV PC,LR        ;EXIT SUB-ROUTINE SR_CPLBIT

;SUB-ROUTINES FOR WRITING BIT '0/1' TO BITPOS IN MEMORY

SR_WRITEBIT

LDR R3,[R0]      ;GET WORD FROM MEMORY INTO R3

MOV R4,#1        ;

MOV R4,R4,LSL R1 ;1<<BITPOS

;ADD UR INSTRUCTION HERE        ;CHECK BIT LEVEL TO WRITE '1' OR '0'

;ADD UR INSTRUCTION HERE       ;IF BIT LEVEL '1' WRITE AS R3|= R4

;ADD UR INSTRUCTION HERE       ;IF BIT LEVEL '0' WRITE IN R3&=~R4

STR R3,[R0]      ;UPDATE WORD BACK INTO MEMORY

MOV PC,LR        ;EXIT SUB-ROUTINE SR_WRITEBIT

;SUB-ROUTINES FOR READING BIT LEVEL OF BITPOS IN MEMORY

SR_READBIT

LDR R2,[R0]      ;GET WORD FROM MEMORY INTO R2

;ADD UR INSTRUCTION HERE   ;R2=R2>>R1

;ADD  UR INSTRUCTION HERE   ;R2=R2 & 1

MOV PC,LR        ;EXIT SUB-ROUTINE SR_READBIT

END