| #include <LPC21xx.h> #include “types.h” #include “defines.h” //define pin functions #define FUN2 1 //define CFGPIN macro #define CFGPIN(WORD,PIN,FUNC) \ WORD=((PIN<16) ? \ ((WORD&~(3<<(PIN*2)))¦(FUNC<<(PIN*2))) : \ ((WORD&~(3<<((PIN-16)*2)))¦(FUNC<<((PIN-16)*2)))) //TD1 CAN1_TX pin no alternate function,default #define TD1_PIN //define p0.25 as RD1(i.e CAN1_RX) #define RD1_PIN_0_25 FUN2 #define PCLK 60000000 //Hz #define BIT_RATE 125000 //Hz #define QUANTA 16 #define BRP (PCLK/(BIT_RATE*QUANTA)) #define SAMPLE_POINT (0.7 * QUANTA) #define TSEG1 ((u32)SAMPLE_POINT-1) #define TSEG2 (QUANTA-(1+TSEG1)) #define SJW ((TSEG2 < 5)?4:(TSEG2-1)) //cfg values for C1BTR sfr #define TSEG1_LVAL (TSEG1-1) #define TSEG2_LVAL (TSEG2-1) #define BRP_LVAL (BRP-1) #define SJW_LVAL (SJW-1) #define SAM 0 //0 or 1 ,sample bus 1 or 3 time(s) #define BTR_LVAL (SAM<<23¦(TSEG2_LVAL<<20)¦ \ (TSEG1_LVAL<<16)¦ \ (SJW_LVAL<<14)¦BRP_LVAL) //C1CMR sfr bit defines #define TR_BIT 0 #define RRB_BIT 2 #define STB1_BIT 5 //C1SR/C1GSR sfr bit defines #define RBS_BIT 0 #define TBS1_BIT 2 #define TCS1_BIT 3 //C1TFI1/C1RFS sfr bit defines #define RTR_BIT 30 #define DLC_BITS 16 void CAN1_Tx(struct CAN_Frame txFrame) { // Checking that the TX buffer is empty while(READBIT(C1GSR,TBS1_BIT)==0); // Cfg Tx ID C1TID1=txFrame.ID; // Cfg RTR & DLC C1TFI1=(txFrame.BFV.RTR<<RTR_BIT)¦ (txFrame.BFV.DLC<<DLC_BITS); //Check whether Data Frame/ // Remote Frame to Transmit if(txFrame.BFV.RTR!=1) { //if data frame,place data into data transmit buffers C1TDA1= txFrame.Data1; /*bytes 1-4 */ C1TDB1= txFrame.Data2; /*bytes 5-8 */ } //Select Tx Buff 1 & Start Xmission C1CMR¦=1<<STB1_BIT¦1<<TR_BIT; //wait until tx complete while(READBIT(C1GSR,TCS1_BIT)==0); } /* can_tx_test.c */ int main(void) { struct CAN_Frame txFrame; Init_CAN1(); txFrame.ID=1; txFrame.RTR=0; txFrame.DLC=1; txFrame.Data1=1; txFrame.Data2=0; while(1) { CAN1_Tx(txFrame); delay_ms(1000); txFrame.Data1=!txFrame.Data1; } } | #define ACC_FILTER_EN 1 #define ACC_FILTER_RAM_BASE_ADDR 0xE0038000 struct CAN_Frame { u32 ID; struct BitField { u32 RTR : 1; u32 DLC : 4; }BFV; u32 Data1,Data2; }; u32 *stdFilter=(u32 *)ACC_FILTER_RAM_BASE_ADDR; void Init_CAN1(void) { //cfg p0.25 as CAN1_RX pin(RD1) using PINSEL1 CFGPIN(PINSEL1,25,RD1_PIN_0_25); //Reset CAN1 controller using C1MOD C1MOD=1; //all received messages are accepted // AFMR=2; #if ACC_FILTER_EN > 0 //Disable the acceptance filters AFMR=1; stdFilter[0]=0x20012002;//allow ids 1 & 2 stdFilter[1]=0x20052006;//allow ids 5 & 6 //set start address of std table SFF_sa=0x00000000; //set start addr of std group table SFF_GRP_sa=8; //set start addr of ext table EFF_sa=8; //set start addr of ext group table EFF_GRP_sa=8; //set end of table address ENDofTable=8; //enable acceptance filters AFMR=0; #endif //Set baud Rate for CAN1 controller using C1BTR C1BTR=BTR_LVAL; //Enable CAN1 controller using C1MOD C1MOD=0; } void CAN1_Rx(struct CAN_Frame *rxFrame) { //wait for CAN frame recv status while(READBIT(C1GSR,RBS_BIT)==0); //read 11-bit CANid of recvd frame. rxFrame->ID=C1RID; //& read & extract data/remote frame status rxFrame->BFV.RTR=(C1RFS>>RTR_BIT)&1; //& extract data length rxFrame->BFV.DLC=(C1RFS>>DLC_BIT)&0x0f; //check if recvd frame is data frame, if(rxFrame->BFV.RTR==0) { //extract data bytes 1-4 rxFrame->Data1=C1RDA; //extract data bytes 5-8 rxFrame->Data2=C1RDB; } // Release receive buffer command SETBIT(C1CMR,RRB_BIT); } /* can_rx_test.c */ #define OPIN 2 int main(void) { //declare can recv object. struct CAN_Frame rxFrame; //cfg can1 controller Init_CAN1(); //cfg p0.2/gpio pin as output pin SETBIT(IODIR0,OPIN); while(1) { //wait for can message CAN1_Rx(&rxFrame); //update recvd status as high/low on OPIN WRITEBIT(IOPIN0,OPIN,rxFrame.Data1); } } |
| --- | --- |
| /* can_debug.ini */ /* Note: A debug script for CAN debugging & simulation for Keil IDE. Can be used for representing a CAN NODE connected to the bus which can sends CAN(Standard Data/Remote) frame . */ DEFINE BUTTON "DF->1","Send4CAN1(1,1,1,1)" DEFINE BUTTON "DF->2","Send4CAN1(1,1,1,0)" DEFINE BUTTON "DF->3","Send4CAN1(1, | DEFINE BUTTON "DF->4","Send4CAN1(1,1,1,0)" func void Send4CAN1 (unsigned id,unsigned rtr,unsigned dlc,unsigned byte0) { CAN1ID = id; CAN1L=dlc; CAN1B0=byte0; CAN1IN = rtr;//1/2/3,4,CMD FOR STD-DF, EXT-DF ,STD- //RF ,EXT-RF CAN1IN=5;//Start Bus Activit } |