| #include <LPC21xx.h> #include “types.h” #include “defines.h” //define pin functions #define FUN2 1 //define CFGPIN macro #define CFGPIN(WORD,PIN,FUNC) \ WORD=((PIN<16) ? \ ((WORD&~(3<<(PIN*2)))¦(FUNC<<(PIN*2))) : \ ((WORD&~(3<<((PIN-16)*2)))¦(FUNC<<((PIN-16)*2)))) //TD1 CAN1_TX pin no alternate function,default #define TD1_PIN //define p0.25 as RD1(i.e CAN1_RX) #define RD1_PIN_0_25 FUN2 #define PCLK 60000000 //Hz #define BIT_RATE 125000 //Hz #define QUANTA 16 #define BRP (PCLK/(BIT_RATE*QUANTA)) #define SAMPLE_POINT (0.7 * QUANTA) #define TSEG1 ((u32)SAMPLE_POINT-1) #define TSEG2 (QUANTA-(1+TSEG1)) #define SJW ((TSEG2 >= 5)?4:(TSEG2-1)) //cfg values for C1BTR sfr #define SAM 0 //0 or 1 ,sample bus 1 or 3 time(s) #define BTR_LVAL (SAM<<23¦ \ ((TSEG2-1)<<20)¦ \ ((TSEG1-1)<<16)¦ \ ((SJW-1)<<14)¦ (BRP-1)) //C1CMR sfr bit defines #define TR_BIT 0 #define RRB_BIT 2 #define STB1_BIT 5 //C1SR/C1GSR sfr bit defines #define RBS_BIT 0 #define TBS1_BIT 2 #define TCS1_BIT 3 //C1TFI1/C1RFS sfr bit defines #define RTR_BIT 30 #define DLC_BITS 16 #if CAN1_INTERRUPTS_EN > 0 void CAN1_Tx_ISR( void )__irq { u16 dummy; CPLBIT( IOPIN0,C1_TX_LEDpin); dummy=C1ICR; VICVectAddr =0; } void CAN1_Rx_ISR( void )__irq { CPLBIT( IOPIN0,C1_RX_LEDpin); //read 11-bit CANid of recvd frame. rxFrame.ID=C1RID; //& read & extract data/remote frame status rxFrame.BFV.RTR=(C1RFS>>RTR_BIT)&1; //& extract data length rxFrame.BFV.DLC=(C1RFS>>DLC_BITS)&0x0f; //check if recvd frame is data frame, if(rxFrame.BFV.RTR==0) { //extract data bytes 1-4 rxFrame.Data1=C1RDA; //extract data bytes 5-8 rxFrame.Data2=C1RDB; } // Release receive buffer command SETBIT(C1CMR,RRB_BIT); VICVectAddr =0; } #endif /* can_rx_test.c */ int main(void) { Init_CAN1(); while(1) { //wait for can message CAN1_Rx(&rxFrame); SETBIT(IODIR0,rxFrame.ID); //update recvd status as high/low on OPIN WRITEBIT(IOPIN0,rxFrame.ID,rxFrame.Data1); } } | #define ACC_FILTER_EN 1 #define ACC_FILTER_RAM_BASE_ADDR 0xE0038000 //tx/rx status LEDs pins #define C1_TX_LEDpin 0 //p0.0 #define C1_RX_LEDpin 1 //p0.1 #define CAN1_INTERRUPTS_EN 1 struct CAN_Frame { u32 ID; struct BitField { u32 RTR : 1; u32 DLC : 4; }BFV; u32 Data1,Data2; }; u32 *stdFilter=(u32 *)ACC_FILTER_RAM_BASE_ADDR; struct CAN_Frame rxFrame; void Init_CAN1(void) { CFGPIN(PINSEL1,25,RD1_PIN_0_25); C1MOD=1; // AFMR=2; #if ACC_FILTER_EN > 0 AFMR=1; stdFilter[0]=0x20002001;//allow ids 0 & 1 stdFilter[1]=0x20042005;//allow ids 4 & 5 SFF_sa=0x00000000; SFF_GRP_sa=8; EFF_sa=8; EFF_GRP_sa=8; ENDofTable=8; AFMR=0; #endif C1BTR=BTR_LVAL; #if CAN1_INTERRUPTS_EN > 0 C1IER = 0; /* disable all interrupt */ C1GSR = 0; /* clear all interrupts VICVectCntl0 = 0x20¦20; VICVectAddr0 = (unsigned)CAN1_Tx_ISR; VICIntEnable = 1<<20; C1IER = 0x01;// Enable the Transmit interrupt VICVectCntl1 = 0x20¦26; VICVectAddr1 = (unsigned)CAN1_Rx_ISR; VICIntEnable¦= 1<<26; VICIntSelect = 0x00000000; C1IER ¦= 0x02; //Enable the Receive interrupt IODIR0¦= 1<<C1_TX_LEDpin¦1<<C1_RX_LEDpin; #endif C1MOD=0; } void CAN1_Tx(struct CAN_Frame txFrame) { // Checking that the TX buffer is empty while(READBIT(C1GSR,TBS1_BIT)==0); // Cfg Tx ID C1TID1=txFrame.ID; // Cfg RTR & DLC C1TFI1=(txFrame.BFV.RTR<<RTR_BIT)¦ (txFrame.BFV.DLC<<DLC_BITS); //Check whether Data Frame/ // Remote Frame to Transmit if(txFrame.BFV.RTR!=1) { //if data frame,place data into data transmit buffers C1TDA1= txFrame.Data1; /*bytes 1-4 */ C1TDB1= txFrame.Data2; /*bytes 5-8 */ } //Select Tx Buff 1 & Start Xmission C1CMR¦=1<<STB1_BIT¦1<<TR_BIT; //wait until tx complete while(READBIT(C1GSR,TCS1_BIT)==0); } /* can_tx_test.c */ int main(void) { struct CAN_Frame txFrame; Init_CAN1(); txFrame.ID=1; txFrame.RTR=0; txFrame.DLC=1; txFrame.Data1=1; txFrame.Data2=0; while(1) { CAN1_Tx(txFrame); delay_ms(1000); txFrame.Data1=!txFrame.Data1; } } |
| --- | --- |

/* can_debug.ini */

/*

Note: A debug script for CAN debugging & simulation for Keil IDE.

can be used for representing a CAN NODE connected to the bus

which	can sends CAN(Standard Data/Remote) frame .

*/

DEFINE BUTTON "DF->0a","Send4CAN1(0,1,1,1)"

DEFINE BUTTON "DF->0b","Send4CAN1(0,1,1,0)"

DEFINE BUTTON "DF->1a","Send4CAN1(1,1,1,1)"

DEFINE BUTTON "DF->1b","Send4CAN1(1,1,1,0)"

DEFINE BUTTON "DF->2a","Send4CAN1(2,1,1,1)"

DEFINE BUTTON "DF->2b","Send4CAN1(2,1,1,0)"

DEFINE BUTTON "DF->3a","Send4CAN1(3,1,1,1)"

DEFINE BUTTON "DF->3b","Send4CAN1(3,1,1,0)"

DEFINE BUTTON "DF->4a","Send4CAN1(4,1,1,1)"

DEFINE BUTTON "DF->4b","Send4CAN1(4,1,1,0)"

DEFINE BUTTON "DF->5a","Send4CAN1(5,1,1,1)"

DEFINE BUTTON "DF->5b","Send4CAN1(5,1,1,0)"

func void Send4CAN1 (unsigned id, unsigned rtr, unsigned dlc, unsigned byte0)

{

CAN1ID = id;	CAN1L=dlc; 	CAN1B0=byte0;

CAN1IN = rtr;//1/2/3,4,CMD FOR STD-DF,STD-RF,EXT-DF,EXT-RF

CAN1IN=5;//Start Bus Activit

}