| #include <LPC21xx.h> #include "types.h" #include "defines.h" #include "delay.h" #define PCLK 60000000 //Hz #define BIT_RATE 125000 //Hz #define QUANTA 16 #define BRP (PCLK/(BIT_RATE*QUANTA)) #define SAMPLE_POINT (0.7 * QUANTA) #define TSEG1 ((int)SAMPLE_POINT-1) #define TSEG2 (QUANTA-(1+TSEG1)) #define SJW ((TSEG2 >= 5)?4:(TSEG2-1)) //cfg values for C1BTR sfr #define SAM 0 //0 or 1 ,sample bus 1 or 3 time(s) #define BTR_LVAL \ (SAM<<23¦((TSEG21)<<20)¦((TSEG1-1)<<16)¦ \ ((SJW-1)<<14)¦(BRP-1)) //PINSEL1 defines #define TD1_PIN //TD1 CAN1_TX no alt-function //p0.25 as RD1(i.e CAN1_RX) #define RD1_PIN 0x00040000 //p0.24 as TD2(i.e CAN2_TX) #define TD2_PIN 0x00010000 //p0.23 as RD2(i.e CAN2_RX) #define RD2_PIN 0x00004000 //cfg value for C1CMR sfr bit defines #define TR_BIT 0 #define RRB_BIT 2 #define STB1_BIT 5 //C1SR/C1GSR sfr bit defines #define RBS_BIT 0 #define TBS1_BIT 2 #define TCS1_BIT 3 //C1TFI1/C1RFS sfr bit defines #define RTR_BIT 30 #define DLC_BITS 16 typedef struct CAN_FRAME { u32 ID; struct BitField { u32 RTR:1; u32 DLC:4; }BFV; u32 Data1,Data2; }CANF; void CAN1_Rx(CANF *rxFrame) { //wait for CAN frame recv status while(READBIT(C1GSR,RBS_BIT)==0); //read 11-bit CANid of recvd frame. rxFrame->ID=C1RID; //& read & extract data/remote frame status rxFrame->BFV.RTR=(C1RFS>>RTR_BIT)&1; //& extract data length rxFrame->BFV.DLC=(C1RFS>>DLC_BITS)&0x0f; //check if recvd frame is data frame, if(rxFrame->BFV.RTR==0) { //extract data bytes 1-4 rxFrame->Data1=C1RDA; //extract data bytes 5-8 rxFrame->Data2=C1RDB; } // Release receive buffer command SETBIT(C1CMR,RRB_BIT); } void Init_CAN2(void) { //p0.23/p0.24 pin cfg as RD2,TD2 PINSEL1¦=RD2_PIN¦TD2_PIN; //Reset CAN2 controller C2MOD=1; //all received messages are accepted AFMR=2; //Set baud Rate for CAN C2BTR=BTR_LVAL; //Enable CAN1 controller C2MOD=0; } void CAN2_Tx(CANF txFrame) { // Checking that the TX buffer is empty while(READBIT(C2GSR,TBS1_BIT)==0); // cfg tx id C2TID1=txFrame.ID; // cfg RTR & DLC C2TFI1=txFrame.BFV.RTR<<30¦txFrame.BFV.DLC<<16; if(txFrame.BFV.RTR!=1) { C2TDA1= txFrame.Data1; /*Transmit data bytes 1-4 */ C2TDB1= txFrame.Data2; /*Transmit data bytes 5-8 */ } //select Tx Buf1 & Start Xmission C2CMR¦=1<<STB1_BIT¦1<<TR_BIT; //wait until tx complete while(READBIT(C2GSR,TCS1_BIT)==0); } | void Init_CAN1(void) { //p0.25 pin cfg as CAN1_RX pin(RD1) PINSEL1¦=RD1_PIN; //Reset CAN1 controller C1MOD=1; //all received messages are accepted AFMR=2; //Set baud Rate for CAN C1BTR=BTR_LVAL; //Enable CAN1 controller C1MOD=0; } void CAN1_Tx(CANF txFrame) { // Checking that the TX buffer is empty while(READBIT(C1GSR,TBS1_BIT)==0); // Cfg Tx ID C1TID1=txFrame.ID; // Cfg RTR & DLC C1TFI1=txFrame.BFV.RTR<<RTR_BIT¦ txFrame.BFV.DLC<<DLC_BITS; //Check whether Data Frame or // Remote Frame to Transmit if(txFrame.BFV.RTR!=1) { //if data frame,place data into data transmit buffers C1TDA1= txFrame.Data1; /*bytes 1-4 */ C1TDB1= txFrame.Data2; /*bytes 5-8 */ } //Select Tx Buff 1 & Start Xmission C1CMR¦=1<<STB1_BIT¦1<<TR_BIT; //wait until tx complete while(READBIT(C1GSR,TCS1_BIT)==0); } void CAN2_Rx(CANF *rxFrame) { //wait for CAN frame recv status while( READBIT(C2GSR,RBS_BIT)==0); //The 11-bit CANid of recvd frame. rxFrame->ID=C2RID; //& read & extract data/remote frame status rxFrame->BFV.RTR=(C2RFS>>RTR_BIT)&1; //& extract data length rxFrame->BFV.DLC=(C2RFS>>DLC_BITS)&0x0f; //check if recvd frame is data frame, if(rxFrame->BFV.RTR==0) { //extract data bytes 1 - 4 rxFrame->Data1=C2RDA; //extract data bytes 5 - 8 rxFrame->Data2=C2RDB; } // Release receive buffer command SETBIT(C2CMR,RRB_BIT); } main() { CANF txFrame={1,0,1,0,0},rxFrame;//tx frame Init_CAN1(); Init_CAN2(); SETBIT(IODIR0,7); txFrame.Data1=1; while(1) { CAN1_Tx(txFrame); CAN2_Rx(&rxFrame); WRITEBIT(IOPIN0,7,rxFrame.Data1); delay_ms(1000); txFrame.Data1=!txFrame.Data1; } } /* can_debug.ini */ signal void recvfromcan1_sendtocan2(void) { while (1) { wwatch(CAN1OUT); CAN2B0=CAN1B0; CAN2B1=CAN1B1; CAN2B2=CAN1B2; CAN2B3=CAN1B3; CAN2B4=CAN1B4; CAN2B5=CAN1B5; CAN2B6=CAN1B6; CAN2B7=CAN1B7; CAN2L=CAN1L; CAN2ID = CAN1ID; CAN2IN = 1; CAN2IN = 5;//Start Bus Activity } } recvfromcan1_sendtocan2() |
| --- | --- |